SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2 - COMPARATIVE ANALYSIS

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO4 | Assessment: | ASSESSMENT TOOL 2-COMPARATIVE ANALYSIS |
| Weightage: | 40% | Total Marks: | 25 |
| CO4 Statement: | Implement hierarchical memory systems by differentiating cache levels (L1, L2, L3), virtual memory with paging, and advanced memory technologies (DRAM, SRAM, NAND Flash, MRAM, RRAM) based on latency, bandwidth, and throughput characteristics. |  |  |
| Student Name: | R.MOHANRAM | Reg.No.: | 192521117 |
| Department: | B.tech IT | Date: | 30/8/26 |

ANNEXURE A

COMPARATIVE ANALYSIS

Q1. Compare L1, L2, and L3 Cache Levels in Terms of Latency, Bandwidth, and Throughput, and Analyze Their Impact on Processor Performance

1. Introduction

Cache memory is a small and high-speed memory located close to the processor. It stores frequently used instructions and data so that the processor does not need to access slower main memory every time.

Modern processors generally use multiple levels of cache memory:

- L1 Cache

- L2 Cache

- L3 Cache

These cache levels differ in terms of:

- Latency

- Bandwidth

- Capacity

- Throughput

- Cost

A hierarchical cache system improves processor performance by providing a balance between high speed and large memory capacity.

2. L1 Cache

L1 cache is the first level of cache memory and is located closest to the CPU core.

Characteristics

- Lowest latency.

- Highest speed.

- Smallest capacity.

- Very high bandwidth.

- Usually dedicated to each processor core.

L1 cache stores the most frequently used instructions and data.

Because it is located very close to the processor, the CPU can access L1 cache with very low delay.

3. L2 Cache

L2 cache is the second level of cache memory.

It is larger than L1 cache but slower.

Characteristics

- Higher latency than L1.

- Larger capacity than L1.

- High bandwidth.

- Stores recently used data.

- May be dedicated to a CPU core.

L2 cache is accessed when the required data is not available in L1 cache.

4. L3 Cache

L3 cache is the third level of cache memory.

It is larger than L1 and L2 cache but has higher latency.

Characteristics

- Larger capacity.

- Higher latency than L1 and L2.

- Usually shared between multiple CPU cores.

- Lower speed than L1 and L2.

- Still much faster than DRAM.

L3 cache reduces the need to access main memory.

5. Comparison Table

| Feature | L1 Cache | L2 Cache | L3 Cache |
| --- | --- | --- | --- |
| Location | Closest to CPU | Near CPU | Shared processor level |
| Latency | Very Low | Low | Higher than L1 and L2 |
| Bandwidth | Very High | High | Moderate to High |
| Capacity | Very Small | Small to Medium | Large |
| Speed | Fastest | Fast | Slower than L1 and L2 |
| Throughput | Very High | High | High for multicore systems |
| Sharing | Usually private | Usually private or dedicated | Usually shared |
| Main Purpose | Frequently used data | Recently used data | Shared and larger working data |

6. Latency Comparison

Latency is the time required to access data.

The latency generally follows:

L1 < L2 < L3 < DRAM

L1 cache provides the lowest latency because it is closest to the CPU.

L2 cache has slightly higher latency but provides more storage capacity.

L3 cache has higher latency than L1 and L2 but is still significantly faster than accessing DRAM.

Therefore, the processor first checks L1 cache, followed by L2 and L3.

7. Bandwidth Comparison

Bandwidth represents the amount of data transferred per unit time.

L1 cache provides very high bandwidth because it serves the CPU directly.

L2 cache also provides high bandwidth but may have slightly lower performance than L1.

L3 cache provides a larger shared memory area and supports data sharing between multiple processor cores.

Therefore, L3 cache is important for multicore processors.

8. Throughput Comparison

Throughput represents the amount of data or instructions processed in a given time.

L1 cache provides the highest immediate performance because of its low latency.

L2 cache improves throughput by reducing L1 cache misses.

L3 cache improves overall processor throughput by reducing the number of accesses to DRAM.

In multicore processors, L3 cache can improve system throughput because multiple CPU cores can access shared data.

9. Impact on Processor Performance

The cache hierarchy improves processor performance in the following ways:

L1 Cache

- Provides the fastest data access.

- Reduces CPU waiting time.

- Improves instruction execution speed.

L2 Cache

- Reduces the effect of L1 cache misses.

- Provides additional high-speed storage.

- Improves processor efficiency.

L3 Cache

- Reduces DRAM access.

- Supports multicore processors.

- Improves overall system throughput.

10. Critical Analysis

A larger cache is not always faster.

Increasing cache capacity can increase access latency.

Therefore, a hierarchy is necessary.

L1 cache is kept small to maintain extremely low latency.

L2 cache provides a balance between capacity and speed.

L3 cache provides larger capacity and supports data sharing.

The hierarchical approach is more effective than using a single large cache.

11. Conclusion

L1, L2, and L3 cache levels have different roles in processor performance.

L1 provides the lowest latency and highest immediate performance. L2 provides additional capacity while maintaining high speed. L3 provides larger shared storage and reduces expensive DRAM accesses.

The combination of L1, L2, and L3 improves latency, bandwidth utilization, and processor throughput.

Q2. Differentiate SRAM and DRAM by Evaluating Their Latency, Bandwidth, and Throughput Characteristics in Cache and Main Memory Design

1. Introduction

SRAM and DRAM are two important semiconductor memory technologies.

They are used for different purposes because they have different:

- Speed

- Latency

- Capacity

- Cost

- Power requirements

- Throughput

SRAM is generally used for cache memory, while DRAM is generally used for main memory.

2. SRAM

SRAM stands for Static Random Access Memory.

SRAM stores data using electronic circuits and does not require continuous refresh while power is available.

Characteristics

- Very low latency.

- Very high speed.

- High bandwidth.

- Expensive.

- Lower storage density.

SRAM is commonly used for:

- L1 Cache

- L2 Cache

- L3 Cache

3. DRAM

DRAM stands for Dynamic Random Access Memory.

DRAM stores data using capacitors.

The stored charge must be refreshed periodically.

Characteristics

- Higher latency than SRAM.

- Large capacity.

- Lower cost per bit.

- High memory density.

- Requires periodic refresh.

DRAM is commonly used as the main memory of computers and servers.

4. Comparison Table

| Feature | SRAM | DRAM |
| --- | --- | --- |
| Full Form | Static Random Access Memory | Dynamic Random Access Memory |
| Latency | Very Low | Higher |
| Speed | Very High | High |
| Bandwidth | Very High | High |
| Throughput | Very High | High |
| Capacity | Small | Large |
| Cost | High | Lower |
| Density | Low | High |
| Refresh Required | No | Yes |
| Power | Higher active area cost | Requires refresh power |
| Main Use | Cache Memory | Main Memory |

5. Latency Comparison

SRAM provides lower latency because its data can be accessed quickly using its internal circuit structure.

DRAM has higher latency because memory access involves activating rows and reading stored charges.

Therefore:

SRAM Latency < DRAM Latency

This is why SRAM is suitable for processor cache.

6. Bandwidth Comparison

SRAM provides very high bandwidth for small and frequently accessed data.

DRAM provides high bandwidth and much larger capacity.

Modern systems use multiple memory channels and advanced DRAM designs to increase bandwidth.

Therefore:

- SRAM is best for extremely fast access.

- DRAM is best for transferring large amounts of data.

7. Throughput Comparison

SRAM improves processor throughput because the CPU can access frequently required data quickly.

DRAM supports large workloads because it provides much greater capacity.

However, frequent DRAM access can reduce CPU throughput because the processor may wait longer for data.

Therefore, cache memory is used to reduce the number of DRAM accesses.

8. SRAM in Cache Design

SRAM is used in cache because cache requires:

- Very low latency.

- Fast access.

- High bandwidth.

The CPU frequently accesses cache memory.

Using DRAM for L1 cache would increase processor access latency.

Therefore, SRAM is more suitable for cache memory.

9. DRAM in Main Memory Design

DRAM is used as main memory because systems require large storage capacity.

Applications, operating systems, and active data require gigabytes of memory.

Using SRAM for all main memory would be too expensive.

Therefore, DRAM provides a better balance between:

- Cost.

- Capacity.

- Performance.

10. Critical Analysis

SRAM is faster but expensive.

DRAM is slower but provides much greater capacity at a lower cost.

Therefore, neither technology can completely replace the other in a typical computer system.

The best design is a hierarchy:

CPU → SRAM Cache → DRAM Main Memory

SRAM improves speed, while DRAM provides capacity.

11. Conclusion

SRAM and DRAM have different characteristics.

SRAM provides very low latency and high throughput, making it suitable for cache memory.

DRAM provides larger capacity and high bandwidth, making it suitable for main memory.

The combination of SRAM and DRAM provides an effective balance between processor performance, memory capacity, and cost.

Q3. Compare Virtual Memory with Paging and Cache Memory Mechanisms in Terms of Access Latency and System Throughput

1. Introduction

Modern computer systems use both cache memory and virtual memory.

Although both improve memory management, they have different purposes.

Cache memory improves processor speed.

Virtual memory allows the operating system to use storage as an extension of physical memory.

Virtual memory generally uses a technique called paging.

2. Cache Memory

Cache memory stores frequently used data close to the processor.

The processor checks:

L1 → L2 → L3 → DRAM

The purpose of cache is to reduce memory access latency.

3. Virtual Memory

Virtual memory allows a program to use more memory than the available physical DRAM.

The operating system divides memory into fixed-size blocks called pages.

When required data is not available in DRAM, the system loads the page from secondary storage.

This is called a page fault.

4. Paging Mechanism

The paging process is:

- CPU requests data.

- Required page is checked in DRAM.

- If unavailable, a page fault occurs.

- Operating system accesses secondary storage.

- Required page is transferred to DRAM.

- Program execution continues.

This process has much higher latency compared with cache access.

5. Comparison Table

| Feature | Cache Memory | Virtual Memory with Paging |
| --- | --- | --- |
| Main Purpose | Reduce access latency | Extend available memory |
| Location | CPU and DRAM | DRAM and secondary storage |
| Access Speed | Very Fast | Slow during page fault |
| Latency | Very Low | High |
| Capacity | Small | Very Large virtual space |
| Managed By | Hardware | Operating System and Hardware |
| Main Problem | Cache Miss | Page Fault |
| Effect on Throughput | Improves throughput | Can reduce throughput during paging |

6. Access Latency Comparison

Cache memory provides low latency.

If data is available in L1 cache, it can be accessed very quickly.

If data is not found, the system checks L2 and L3.

Virtual memory paging has much higher latency.

When a page fault occurs, data must be transferred from storage to DRAM.

Therefore:

Cache Access Latency << Page Fault Latency

7. Throughput Comparison

Cache improves system throughput by reducing CPU waiting time.

More instructions can be processed because data is available quickly.

Virtual memory improves system flexibility by allowing large applications to run.

However, excessive paging reduces throughput because the processor waits while pages are transferred from storage.

8. Cache Miss vs Page Fault

A cache miss occurs when data is not found in the cache.

The system may then access DRAM.

A page fault occurs when the required page is not available in DRAM.

The system may need to access secondary storage.

Therefore, a page fault generally has a much greater performance impact.

9. Impact on System Performance

Cache Memory

Cache improves:

- CPU performance.

- Instruction execution.

- Application response time.

- System throughput.

Virtual Memory

Virtual memory improves:

- Memory capacity.

- Application flexibility.

- Ability to run large programs.

However, excessive paging can:

- Increase latency.

- Reduce throughput.

- Cause system slowdown.

10. Critical Analysis

Cache and virtual memory should not be considered as alternatives.

They solve different problems.

Cache memory is designed for speed.

Virtual memory is designed for memory capacity and flexibility.

A well-designed system should:

- Maximize cache hits.

- Provide sufficient DRAM.

- Minimize page faults.

This produces better overall performance.

11. Conclusion

Cache memory and virtual memory have different purposes.

Cache memory reduces processor access latency and improves throughput.

Virtual memory allows systems to run larger programs but page faults can introduce high latency.

The best system uses efficient cache hierarchy and sufficient DRAM to minimize paging.

Q4. Analyze the Differences Between NAND Flash and DRAM with Respect to Latency, Bandwidth, and Suitability for Hierarchical Memory Systems

1. Introduction

NAND Flash and DRAM are important memory technologies used in modern computing systems.

However, they have very different characteristics.

DRAM is mainly used for active main memory.

NAND Flash is mainly used for non-volatile storage.

Both technologies can be combined in a hierarchical memory system.

2. NAND Flash

NAND Flash is a non-volatile memory technology.

It retains data even when power is removed.

It is commonly used in:

- SSDs.

- Smartphones.

- Memory cards.

- USB drives.

Characteristics

- Large capacity.

- Non-volatile.

- Lower cost per bit.

- Higher access latency than DRAM.

3. DRAM

DRAM is volatile memory.

It stores active data and program information while the system is running.

It is commonly used as:

- Main memory.

- System memory.

- Server memory.

Characteristics

- Low latency compared with NAND Flash.

- High bandwidth.

- Large capacity.

- Requires continuous power and refresh.

4. Comparison Table

| Feature | NAND Flash | DRAM |
| --- | --- | --- |
| Volatility | Non-Volatile | Volatile |
| Latency | Higher | Lower |
| Bandwidth | Lower than active DRAM memory | High |
| Capacity | Very Large | Large |
| Cost per Bit | Lower | Higher |
| Access Method | Block/Page-Based | Random Access |
| Power Off Data | Retained | Lost |
| Main Use | Storage | Main Memory |
| Speed | Moderate | High |

5. Latency Comparison

DRAM provides lower access latency.

It is designed to supply data quickly to the processor.

NAND Flash has higher latency because data is accessed through storage controllers and flash memory operations.

Therefore:

DRAM Latency < NAND Flash Latency

This is why NAND Flash is not normally used as direct main memory for high-speed processing.

6. Bandwidth Comparison

DRAM provides high bandwidth for active processing.

Modern processors and GPUs require high-speed memory access.

NAND Flash provides high storage capacity but generally has higher access overhead.

Therefore:

- DRAM is suitable for active workloads.

- NAND Flash is suitable for long-term storage.

7. Throughput Comparison

DRAM supports high throughput for applications because it can provide data continuously to the processor.

NAND Flash storage throughput is useful for loading applications and files but is generally not intended to replace DRAM for active memory access.

Therefore, DRAM is better for continuous processing workloads.

8. Role in Hierarchical Memory Systems

A typical memory hierarchy is:

CPU

↓

L1 Cache

↓

L2 Cache

↓

L3 Cache

↓

DRAM

↓

NAND Flash Storage

Each level provides a different balance between speed and capacity.

9. DRAM Role

DRAM stores:

- Running applications.

- Active data.

- Program instructions.

- Temporary calculations.

DRAM acts as the main working memory.

10. NAND Flash Role

NAND Flash stores:

- Operating systems.

- Applications.

- Documents.

- Media files.

- Large datasets.

It provides permanent storage.

11. Critical Analysis

DRAM and NAND Flash are complementary technologies.

DRAM provides speed but loses data when power is removed.

NAND Flash provides persistent storage but has higher latency.

Using only DRAM would be expensive and would not provide permanent data storage.

Using only NAND Flash would reduce active processing performance.

Therefore, combining both in a memory hierarchy provides the best balance.

12. Conclusion

DRAM provides lower latency and higher bandwidth for active processing.

NAND Flash provides large-capacity and non-volatile storage.

In a hierarchical memory system, DRAM acts as fast main memory while NAND Flash acts as slower but persistent storage.

The combination improves both performance and storage capacity.

Q5. Compare MRAM and RRAM Technologies by Examining Their Performance in Terms of Access Time, Throughput, and Energy Efficiency

1. Introduction

MRAM and RRAM are advanced non-volatile memory technologies.

They are being developed to provide improvements in:

- Speed.

- Power efficiency.

- Data retention.

- Memory density.

Both technologies can retain data even when power is removed.

However, they use different physical mechanisms.

2. MRAM

MRAM stands for Magnetoresistive Random Access Memory.

MRAM stores data using magnetic states.

It is a non-volatile memory technology.

Characteristics

- Fast access.

- Non-volatile.

- Good endurance.

- Low standby power.

- Suitable for reliable systems.

Possible applications include:

- Embedded systems.

- Automotive systems.

- Industrial systems.

- Persistent memory.

3. RRAM

RRAM stands for Resistive Random Access Memory.

RRAM stores information by changing the electrical resistance of a material.

It is also non-volatile.

Characteristics

- Fast access potential.

- Low power consumption.

- High storage density.

- Suitable for future AI and edge computing systems.

Possible applications include:

- AI accelerators.

- Edge devices.

- Low-power systems.

- Neuromorphic computing.

4. Comparison Table

| Feature | MRAM | RRAM |
| --- | --- | --- |
| Full Form | Magnetoresistive RAM | Resistive RAM |
| Storage Method | Magnetic State | Resistance Change |
| Volatility | Non-Volatile | Non-Volatile |
| Access Time | Fast | Fast Potential |
| Throughput | High | High Potential |
| Energy Efficiency | Good | Very High Potential |
| Standby Power | Very Low | Very Low |
| Endurance | High | Can vary |
| Density | Moderate | High Potential |
| Main Applications | Reliable Embedded Systems | AI and Low-Power Computing |

5. Access Time Comparison

Both MRAM and RRAM can provide faster access than traditional non-volatile storage technologies.

MRAM provides reliable and fast access.

RRAM also has the potential for low-latency access.

However, performance can depend on the specific implementation.

MRAM is generally valued for predictable and reliable performance.

RRAM is attractive because of its potential for very compact and energy-efficient memory structures.

6. Throughput Comparison

Throughput depends on how much data can be read or written in a given time.

MRAM can provide high throughput because it supports fast memory operations.

RRAM also has high throughput potential and can support parallel memory structures.

For future AI applications, RRAM may be useful because memory and computation can potentially be placed closer together.

7. Energy Efficiency Comparison

Both technologies are non-volatile.

Therefore, they do not require continuous power to retain data.

MRAM provides low standby power and strong reliability.

RRAM has strong potential for very low power operation because resistance states can be retained without continuous refresh.

This makes RRAM attractive for:

- Edge devices.

- IoT systems.

- AI accelerators.

8. Reliability Comparison

MRAM is known for good endurance and reliability.

This makes it useful for systems requiring frequent memory operations.

Examples include:

- Automotive systems.

- Industrial systems.

- Embedded systems.

RRAM performance and endurance can vary depending on the material and device design.

Therefore, reliability and manufacturing consistency remain important considerations.

9. Suitability for Applications

MRAM

MRAM is suitable for:

- Automotive systems.

- Safety-critical systems.

- Embedded applications.

- Persistent memory.

Its main advantages are:

- Reliability.

- Fast access.

- High endurance.

- Non-volatility.

RRAM

RRAM is suitable for:

- AI systems.

- Edge devices.

- Low-power computing.

- Future neuromorphic systems.

Its main advantages are:

- High density potential.

- Low power potential.

- Non-volatility.

- Potential support for AI processing.

10. Critical Analysis

MRAM and RRAM are both promising alternatives to traditional memory technologies.

MRAM provides strong reliability and endurance.

RRAM provides strong potential for high-density and energy-efficient memory.

The selection depends on the application.

For safety-critical systems, MRAM may be preferred because of reliability.

For future AI and low-power systems, RRAM may provide important advantages.

Therefore, both technologies can play different roles in future hierarchical memory systems.

11. Conclusion

MRAM and RRAM are advanced non-volatile memory technologies.

MRAM provides fast access, high reliability, and good endurance.

RRAM provides potential advantages in energy efficiency, memory density, and AI applications.

MRAM is more suitable for reliable and safety-critical applications, while RRAM has strong potential for future AI and low-power computing systems.

Both technologies can contribute to the development of advanced hierarchical memory systems.

OVERALL COMPARATIVE SUMMARY

| Technology | Latency | Bandwidth | Throughput | Capacity | Main Use |
| --- | --- | --- | --- | --- | --- |
| L1 Cache | Very Low | Very High | Very High | Very Small | Frequently Used Data |
| L2 Cache | Low | High | High | Small | Recently Used Data |
| L3 Cache | Moderate | High | High | Larger | Shared Processor Data |
| SRAM | Very Low | Very High | Very High | Small | Cache Memory |
| DRAM | Low to Moderate | High | High | Large | Main Memory |  |
| NAND Flash | High | Moderate | Moderate | Very Large | Permanent Storage |
| MRAM | Low | High | High | Medium | Persistent Reliable Memory |
| RRAM | Low Potential | High Potential | High Potential | High Potential | Future AI and Low-Power Memory |

OVERALL CONCLUSION

Hierarchical memory systems are essential for modern computer architecture because no single memory technology can provide the lowest latency, highest bandwidth, largest capacity, lowest cost, and lowest power consumption at the same time.

Cache levels L1, L2, and L3 reduce processor memory access latency. SRAM provides high-speed cache memory, while DRAM provides large-capacity main memory. Virtual memory extends the available memory space but excessive paging can reduce system throughput.

NAND Flash provides non-volatile and large-capacity storage, while advanced technologies such as MRAM and RRAM provide new possibilities for persistent, low-power, and high-performance memory systems.

Therefore, modern computer systems use a hierarchical combination of different memory technologies to balance:

- Latency

- Bandwidth

- Throughput

- Capacity

- Cost

- Power Consumption

- Reliability

This comparative analysis demonstrates that selecting the correct memory technology for each level of the memory hierarchy is essential for achieving high overall system performance.

Code of Conduct:

I _R.MOHANRAM/192521117_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: mohanram

MARKS ALLOCATION

|  | Scope of Items(20%) | Comparison Depth(25%) | Use of Evidence(20%) | Critical Thinking(20%) | Clarity of Presentation (15%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Scope of Items | 20% | Selects highly relevant items with clear scope and justification | Selects appropriate items with minor gaps | Selection is limited or partially relevant | Items are unclear or not relevant |
| Comparison Depth | 25% | Provides detailed and comprehensive comparison across multiple aspects | Provides adequate comparison with some depth | Limited comparison with few aspects | Very minimal or no comparison |
| Use of Evidence | 20% | Supports comparison with accurate data, examples, or references | Uses some supporting evidence with minor gaps | Limited or weak supporting evidence | No supporting evidence provided |
| Critical Thinking | 20% | Demonstrates strong critical thinking with clear interpretation and reasoning | Shows reasonable interpretation with minor gaps | Basic interpretation; lacks depth | No meaningful interpretation |
| Clarity of Presentation | 15% | Well-organized, clear, and logically structured presentation | Generally clear with minor issues | Somewhat unclear or poorly structured | Disorganized and difficult to understand |