DESIGN AND PERFORMANCE ANALYSIS OF A PIPELINE PROCESSOR WITH DATA HAZARD DETECTION, FORWARDING, AND MEMORY HIERARCHY

CSA1205 – COMPUTER ARCHITECTURE

COMPLETE ASSIGNMENT IMPLEMENTATION REPORT

| Report Information | Details |
| --- | --- |
| Course Code | CSA1205 |
| Course Title | Computer Architecture |
| Assignment Title | Design and Performance Analysis of a Pipeline Processor with Data Hazard Detection, Forwarding, and Memory Hierarchy |
| Course Outcomes | CO3 and CO4 |
| Implementation | 5-Stage RISC Pipeline Simulator with Hazard Detection, Forwarding and Analytical L1 Miss Modeling |
| SDG Mapping | SDG 9 – Industry, Innovation and Infrastructure |
| Prepared By | MOHANRAM R |

# Plagiarism Report / Similarity Declaration

A plagiarism or similarity report may be inserted on this page after the final document has been checked using the institution-approved similarity checking tool.

This implementation report documents the submitted project source files, including the C++ hardware-style pipeline simulator, Flask simulation backend, Verilog forwarding unit, Verilog hazard detection unit, and supporting project documentation.

# 1. Introduction

Modern processors improve performance by overlapping the execution of multiple instructions. A classic implementation is the five-stage RISC pipeline consisting of Instruction Fetch (IF), Instruction Decode/Register Read (ID), Execute (EX), Memory Access (MEM), and Write Back (WB). Although pipelining improves instruction throughput, dependencies between instructions can cause data hazards. The most relevant hazard for this assignment is the Read-After-Write (RAW) dependency, where a consumer instruction attempts to read a register before the producing instruction has made the required result available.

The submitted project implements and simulates a five-stage RISC pipeline using a hardware-oriented C++ model. The benchmark program contains one load instruction followed immediately by a dependent arithmetic instruction and then a chain of dependent ALU instructions. The implementation therefore demonstrates two important mechanisms: a hazard detection unit for the unavoidable load-use dependency and a forwarding unit for bypassing ALU results from later pipeline stages back to the EX stage.

The project also includes a Flask backend that returns a cycle trace and performance metrics in JSON form for a web-based presentation layer. The simulation tracks pipeline state cycle by cycle and records stall/bubble events and forwarding events. The C++ implementation models instruction memory, data memory, a 32-register register file, inter-stage pipeline registers, control signals, forwarding multiplexer selections, retirement count, and execution-cycle metrics.

For memory hierarchy performance, the submitted implementation uses an analytical L1 cache miss model. It assumes a 10% L1 miss rate and an 8-cycle miss penalty for the benchmark's load activity. The implementation computes an expected execution time and resulting CPI under this miss model. Cache size, mapping technique, and replacement policy are not encoded in the submitted source files; therefore, where such parameters are discussed later in this report, they are explicitly presented as a proposed design configuration for satisfying the analytical cache-design requirement rather than as implemented cache-controller behavior.

# 2. Deliverable Mapping

| Deliverable | Implementation / Report Area |
| --- | --- |
| 1 | Working pipeline processor implementation and project architecture |
| 2 | Register-transfer modelling and five-stage pipeline timing |
| 3 | RAW dependency identification in the benchmark instruction sequence |
| 4 | Load-use hazard detection unit and generated control signals |
| 5 | EX/MEM-to-EX and MEM/WB-to-EX forwarding paths |
| 6 | Minimal one-cycle load-use stall and bubble injection |
| 7 | MEM-stage memory access and analytical L1 cache-miss model |
| 8 | CPI, speedup, and pipeline efficiency analysis |
| 9 | Generalization to longer programs and hit-rate/dependency trade-offs |
| 10 | Complete end-to-end execution demonstration and implementation results |
| Reflection | Design decisions, SDG 9 relevance, challenges and learning outcomes |

The following sections use the repeated structure Purpose, Design/Logic, Pseudocode or Algorithm, Sample Input/Trace, and Result/Observation so that the report follows a clear deliverable-oriented format.

# Deliverable 1: Working Pipeline Processor Implementation

## Purpose

The purpose of the working implementation is to provide an executable model of a five-stage RISC processor pipeline and demonstrate how the processor responds to dependent instructions. The implementation combines instruction representation, pipeline registers, hazard detection, forwarding, memory access, cycle tracing, and performance metrics.

## Submitted Project Structure

| File / Component | Role in the Implementation |
| --- | --- |
| app/app.py | Flask backend and JSON cycle-trace simulation for the web interface |
| src/pipeline_sim.cpp | Main hardware-oriented C++ five-stage pipeline simulator |
| src/forwarding_unit.v | Combinational forwarding logic for ALU operands A and B |
| src/hazard_unit.v | Combinational load-use hazard detection interface |
| build/pipeline_sim.exe | Compiled Windows executable supplied with the project |
| docs/Architecture_Report.docx | Supporting report placeholder in submitted project package |
| docs/Pipeline_Timing_Matrix.txt | Timing-matrix placeholder in submitted project package |
| README.md | Project overview and build instructions |

## Implementation Architecture

Instruction Program
       |
       v
+------+-----+    +---------+    +---------+    +---------+    +---------+
|  IF Stage  | -> | ID Stage| -> | EX Stage| -> |MEM Stage| -> | WB Stage|
+------+-----+    +---------+    +---------+    +---------+    +---------+
       |               |              ^                |              |
       |               |              |                |              |
       |         Hazard Detection     |          Data Memory          |
       |          PC/IF-ID Stall      |                                |
       |               |         +----+----------------------+         |
       |               +-------->| Forwarding Unit           |---------+
       |                         | EX/MEM -> EX             |
       +------------------------>| MEM/WB -> EX             |
                                 +--------------------------+

## Core Hardware Data Structures

The C++ simulator defines an Instruction structure containing instruction ID, assembly text, opcode, destination register, and two source registers. It also defines IF/ID, ID/EX, EX/MEM, and MEM/WB pipeline-register structures. The pipeline object contains instruction memory, a 1024-element data memory, a 32-register register file, program counter, cycle counter, stall counter, retirement counter, hazard-control signals, and forwarding multiplexer select signals.

## High-Level Pseudocode

START
  Load benchmark instruction sequence
  Initialize register file and data memory
  WHILE all instructions are not retired
      Perform WB stage
      Perform MEM stage
      Evaluate forwarding unit
      Perform EX stage using forwarded operands when selected
      Evaluate load-use hazard detection
      IF hazard exists
          freeze PC
          freeze IF/ID
          inject bubble into ID/EX
      ELSE
          perform ID/register read
          perform IF
      END IF
      Transfer next-state values into pipeline registers
      print cycle trace and hardware notes
  END WHILE
  compute CPI and analytical cache-miss metrics
STOP

## Result / Observation

The compiled C++ implementation successfully executes the six-instruction benchmark and terminates after 11 cycles. The runtime trace reports one unavoidable load-use stall and multiple forwarding events. The resulting Scenario B CPI is 11/6 = 1.83, and the analytical 10% L1 miss model produces an expected CPI of approximately 1.97.

# Deliverable 2: Register Transfer Modelling and Five-Stage Pipeline Timing

## Purpose

This deliverable models the movement of instructions and data through the IF, ID, EX, MEM, and WB stages. The submitted C++ simulator explicitly maintains inter-stage register structures corresponding to these pipeline boundaries.

## Five-Stage Operation

| Stage | Operation in Submitted Implementation |
| --- | --- |
| IF | Fetch next instruction from instruction vector and update IF/ID register |
| ID | Read source values from the 32-register register file and generate control fields |
| EX | Select forwarded or register-file operands and compute ALU result / load address |
| MEM | For load, read data memory; otherwise pass ALU result toward write-back |
| WB | Write final result to destination register when RegWrite is enabled and retire instruction |

## Register Transfer Notation

IF:
    IF/ID.IR <- IMEM[PC]
    PC <- PC + 1

ID:
    ID/EX.A <- Reg[IF/ID.rs1]
    ID/EX.B <- Reg[IF/ID.rs2]
    ID/EX.rd <- IF/ID.rd
    ID/EX.RegWrite <- (rd != 0)
    ID/EX.MemRead <- (opcode == LOAD)

EX:
    ALU_A <- MUX(RegA, EX/MEM.ALUResult, MEM/WB.FinalValue)
    ALU_B <- MUX(RegB, EX/MEM.ALUResult, MEM/WB.FinalValue)
    EX/MEM.ALUResult <- operation(ALU_A, ALU_B)

MEM:
    IF MemRead = 1
        MEM/WB.FinalValue <- DMEM[EX/MEM.ALUResult mod 1024]
    ELSE
        MEM/WB.FinalValue <- EX/MEM.ALUResult

WB:
    IF MEM/WB.RegWrite = 1 and MEM/WB.rd != 0
        Reg[MEM/WB.rd] <- MEM/WB.FinalValue

## Benchmark Instruction Sequence

| ID | Instruction | Destination | Sources | Dependency Role |
| --- | --- | --- | --- | --- |
| I1 | LW R1, 0(R2) | R1 | R2 | Produces loaded value |
| I2 | ADD R4, R1, R5 | R4 | R1, R5 | Consumes I1 result |
| I3 | SUB R6, R4, R7 | R6 | R4, R7 | Consumes I2 result |
| I4 | AND R8, R6, R9 | R8 | R6, R9 | Consumes I3 result |
| I5 | OR R10, R8, R11 | R10 | R8, R11 | Consumes I4 result |
| I6 | ADD R12, R10, R13 | R12 | R10, R13 | Consumes I5 result |

## Actual Runtime Timing Trace

| Cycle | IF | ID | EX | MEM | WB | Hardware Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | I1 | - | - | - | - |  |
| 2 | I2 | I1 | - | - | - |  |
| 3 | I2 | - | I1 | - | - | STALL (Load-Use Hazard Detected) |
| 4 | I3 | I2 | - | I1 | RETIRE |  |
| 5 | I4 | I3 | I2 | - | - | Forwarding: MEM/WB -> A |
| 6 | I5 | I4 | I3 | I2 | RETIRE | Forwarding: EX/MEM -> A |
| 7 | I6 | I5 | I4 | I3 | RETIRE | Forwarding: EX/MEM -> A |
| 8 | - | I6 | I5 | I4 | RETIRE | Forwarding: EX/MEM -> A |
| 9 | - | - | I6 | I5 | RETIRE | Forwarding: EX/MEM -> A |
| 10 | - | - | - | I6 | RETIRE |  |
| 11 | - | - | - | - | - |  |

## Result / Observation

The cycle trace shows normal overlap after pipeline fill, one bubble caused by the load-use dependency, and subsequent dependency resolution through forwarding. The benchmark completes in 11 cycles.

# Deliverable 3: RAW Data Hazard Identification

## Purpose

A RAW hazard occurs when a later instruction reads a register that an earlier instruction will write. The benchmark was intentionally constructed as a dependency chain so that hazard handling can be observed at each stage.

## RAW Dependency Analysis

| Producer | Consumer | Register | Hazard Type | Resolution in Implementation |
| --- | --- | --- | --- | --- |
| I1: LW R1,... | I2: ADD R4,R1,R5 | R1 | Load-use RAW | One-cycle stall/bubble, then data becomes available |
| I2: ADD R4,... | I3: SUB R6,R4,R7 | R4 | ALU RAW | Forwarding |
| I3: SUB R6,... | I4: AND R8,R6,R9 | R6 | ALU RAW | Forwarding |
| I4: AND R8,... | I5: OR R10,R8,R11 | R8 | ALU RAW | Forwarding |
| I5: OR R10,... | I6: ADD R12,R10,R13 | R10 | ALU RAW | Forwarding |

## Hazard Interpretation

The first dependency is special because I1 is a load. The value loaded from data memory is not available early enough for the immediately following instruction's EX stage. Therefore, forwarding alone is insufficient, and the hazard detection logic inserts one bubble. The remaining dependencies are ALU-to-ALU dependencies, and the implementation's forwarding logic can provide the needed operand from EX/MEM or MEM/WB.

## Pseudocode

FOR each instruction entering the pipeline
    Compare consumer source registers with producer destination registers
    IF producer is a load in ID/EX AND destination matches IF/ID source
        classify as load-use RAW
        stall pipeline for one cycle
    ELSE IF EX/MEM destination matches ID/EX source
        select EX/MEM forwarding
    ELSE IF MEM/WB destination matches ID/EX source
        select MEM/WB forwarding
    ELSE
        use register-file operands
END FOR

## Result / Observation

Five RAW dependencies exist in the benchmark. One is a load-use hazard requiring a stall; the remaining four are handled by forwarding.

# Deliverable 4: Hazard Detection Unit Design

## Purpose

The hazard detection unit identifies the load-use condition and generates control signals that temporarily stop instruction fetch and decoding while inserting a bubble into the execution path.

## Detection Condition

load_use_hazard =
    ID_EX.valid
    AND ID_EX.mem_read
    AND (ID_EX.rd != 0)
    AND IF_ID.valid
    AND ((ID_EX.rd == IF_ID.rs1) OR
         (ID_EX.rd == IF_ID.rs2))

## Generated Control Signals

| Signal | Hazard Present | Normal Operation | Function |
| --- | --- | --- | --- |
| pc_write | 0 | 1 | Freeze or advance program counter |
| if_id_write | 0 | 1 | Freeze or update IF/ID register |
| inject_bubble / control_mux_zero | 1 | 0 | Insert bubble / zero control into next execution path |

## Hazard Detection Block Diagram

ID/EX
            +------------------+
            | MemRead          |
            | rd               |
            +--------+---------+
                     |
                     | compare rd with
                     v
                +----------+
IF/ID.rs1 ------>|          |----> pc_write = 0
IF/ID.rs2 ------>| Hazard   |----> if_id_write = 0
                 | Detector |----> bubble/control zero = 1
                 +----------+
                     |
                No Match
                     v
          Normal pipeline operation

## Operational Pseudocode

IF ID_EX.MemRead = 1 AND ID_EX.rd != 0 AND
   (ID_EX.rd = IF_ID.rs1 OR ID_EX.rd = IF_ID.rs2)
THEN
    PCWrite <- 0
    IF_ID_Write <- 0
    InjectBubble <- 1
ELSE
    PCWrite <- 1
    IF_ID_Write <- 1
    InjectBubble <- 0
END IF

## Result / Observation

During the benchmark execution, the unit detects the dependency between I1 and I2 and generates a single stall/bubble event. The runtime trace reports the event at cycle 3 as 'STALL (Load-Use Hazard Detected)'.

# Deliverable 5: Forwarding / Bypass Path Design

## Purpose

The forwarding unit reduces stalls by bypassing a result directly from a later pipeline stage to the operand input of the EX-stage ALU. The submitted Verilog and C++ implementations support separate forwarding decisions for operand A and operand B.

## Forwarding Select Encoding

| Select | Source | Meaning |
| --- | --- | --- |
| 00 | Register File | No forwarding required |
| 10 | EX/MEM | Forward most recent ALU result |
| 01 | MEM/WB | Forward value available at write-back boundary |

## Forwarding Logic

IF EX_MEM.RegWrite AND EX_MEM.rd != 0 AND
   EX_MEM.rd == ID_EX.rs1
    ForwardA <- EX/MEM
ELSE IF MEM_WB.RegWrite AND MEM_WB.rd != 0 AND
        MEM_WB.rd == ID_EX.rs1
    ForwardA <- MEM/WB
ELSE
    ForwardA <- RegisterFile

Apply the same priority logic independently for ID_EX.rs2 -> ForwardB.

## Forwarding Path Diagram

+----------------+
                      |   EX/MEM       |
                      | ALU Result, rd |
                      +-------+--------+
                              |
                              | EX/MEM -> EX
                              v
Register File -----> [ Forwarding MUX ] -----> ALU Operand A/B
                              ^
                              |
                              | MEM/WB -> EX
                      +-------+--------+
                      |   MEM/WB       |
                      | Final Value,rd |
                      +----------------+

## Observed Forwarding Events

| Cycle | Dependent EX Instruction | Observed Note |
| --- | --- | --- |
| 5 | I2 | Forwarding from MEM/WB to operand A |
| 6 | I3 | Forwarding from EX/MEM to operand A |
| 7 | I4 | Forwarding from EX/MEM to operand A |
| 8 | I5 | Forwarding from EX/MEM to operand A |
| 9 | I6 | Forwarding from EX/MEM to operand A |

## Result / Observation

Forwarding eliminates additional stalls for the dependent ALU chain. Priority is given to EX/MEM because it contains the most recent producer result when both potential matches exist.

# Deliverable 6: Load-Use Hazard and Minimal-Stall Solution

## Purpose

This deliverable demonstrates the dependency that forwarding cannot fully resolve when a load instruction is immediately followed by an instruction requiring the loaded value.

## Problem Sequence

I1: LW  R1, 0(R2)
I2: ADD R4, R1, R5

I2 requires R1, but the loaded value is obtained through the MEM stage.
Therefore, the pipeline must delay I2 for one cycle.

## Minimal-Stall Timing

| Cycle | I1 | I2 | Action |
| --- | --- | --- | --- |
| 1 | IF | - | Fetch load |
| 2 | ID | IF | Decode load and fetch consumer |
| 3 | EX | ID frozen | Load-use detected; inject one bubble |
| 4 | MEM | ID/EX | Load value progresses; consumer resumes |
| 5 | WB | EX | Consumer executes with value available through pipeline forwarding path |

## Control Effect

PC is frozen for one cycle so the next instruction address is not advanced.

IF/ID is frozen so the dependent instruction remains available for later decoding.

A bubble is inserted into the execution path.

After one cycle, normal pipeline movement resumes.

## Result / Observation

The submitted implementation records exactly one load-use stall. This is the minimum-stall strategy for the benchmark's immediate load-use dependency.

# Deliverable 7: Memory Hierarchy and Cache Design for the MEM Stage

## Purpose

The MEM stage in the submitted C++ simulator accesses a 1024-element data memory. For a load instruction, the address computed in EX is used to read data memory and place the resulting value into the MEM/WB register. The submitted project additionally models the performance impact of L1 cache misses analytically.

## Implemented Memory Access

IF EX_MEM.mem_read = true
    MEM_WB.final_val <- DMEM[EX_MEM.alu_result mod 1024]
ELSE
    MEM_WB.final_val <- EX_MEM.alu_result
END IF

## Implemented Analytical Cache Parameters

| Parameter | Value Used in Submitted Performance Model |
| --- | --- |
| L1 miss rate | 10% (0.10) |
| Miss penalty | 8 cycles |
| Load instructions in benchmark | 1 |
| Expected extra cycles | 1 × 0.10 × 8 = 0.8 cycles |
| Expected total cycles | 11 + 0.8 = 11.8 cycles |

## Proposed Cache Configuration for Design Documentation

The source files do not implement cache size, associativity/mapping, or replacement policy. To complete the assignment's required cache-design documentation without misrepresenting the source, the following is proposed as a design-level configuration:

| Design Parameter | Proposed Configuration | Justification |
| --- | --- | --- |
| Cache Level | L1 Data Cache | Directly services the MEM-stage load/store path |
| Capacity | 32 KB | Small enough for low access latency while providing useful locality |
| Mapping | 2-way set associative | Reduces direct-mapped conflict misses without high lookup complexity |
| Replacement | LRU or pseudo-LRU | Provides locality-aware replacement for small associativity |
| Hit latency | 1 cycle assumption | Suitable for pipeline performance analysis |
| Miss penalty | 8 cycles | Matches the submitted analytical implementation |

## Cache Interaction Diagram

EX address
   |
   v
+-----------+     Hit      +----------------+
| L1 Cache  | -----------> | MEM/WB Result  |
+-----------+              +----------------+
     |
     | Miss
     v
Miss handling / lower memory
     |
     +----> add analytical 8-cycle penalty in performance model

## Result / Observation

The implemented simulator models data-memory access and analytical L1 miss penalties. The proposed 32 KB, 2-way set-associative cache is a report-level architectural design assumption; it is not claimed to be an implemented cache controller in the submitted code.

# Deliverable 8: Combined Pipeline and Memory Performance Analysis

## Purpose

This section evaluates execution performance using the implemented cycle count and the analytical cache-miss model. The benchmark contains six instructions and completes in 11 cycles in the forwarding configuration with one unavoidable load-use bubble.

## Scenario A: Stall-Only Baseline

The submitted runtime uses 20 cycles as the reference baseline for the speedup comparison. Therefore, for consistency with the implementation metrics:

N = 6 instructions
Total cycles (baseline reference) = 20
CPI_stall-only = 20 / 6 = 3.33

## Scenario B: Full Forwarding with Cache Hits

Total cycles = 11
CPI_forwarding = 11 / 6 = 1.83

## Scenario C: Full Forwarding with 10% L1 Miss Rate

Expected extra cycles = Loads × Miss Rate × Miss Penalty
                      = 1 × 0.10 × 8
                      = 0.8

Expected cycles = 11 + 0.8 = 11.8
CPI_cache = 11.8 / 6 = 1.97

## Speedup

Speedup = Baseline cycles / Improved expected cycles
        = 20 / 11.8
        = 1.69 ×

## Pipeline Efficiency

Using ideal pipelined CPI = 1 as the reference, an instruction-level pipeline efficiency indicator can be expressed as 1 / CPI. This yields approximately 30.0% for the 3.33 baseline, 54.6% for forwarding with all hits, and 50.8% for forwarding with the expected cache-miss cost. The metric is an analytical efficiency indicator rather than a separate value printed by the submitted executable.

| Scenario | Cycles | CPI | Speedup vs 20-cycle Baseline | Efficiency Indicator (1/CPI) |
| --- | --- | --- | --- | --- |
| A. Stall-only baseline | 20.0 | 3.33 | 1.00× | 30.0% |
| B. Forwarding + cache hits | 11.0 | 1.83 | 1.82× | 54.6% |
| C. Forwarding + 10% L1 miss model | 11.8 expected | 1.97 | 1.69× | 50.8% |

## Result / Observation

Forwarding substantially reduces the cycle cost of the dependency chain. The analytical cache-miss model reduces part of that gain because a memory miss introduces additional expected stall cycles.

# Deliverable 9: Generalized Performance Analysis

## Purpose

For a longer representative program, total performance can be estimated by adding the base pipelined execution cost, unavoidable dependency stalls, and memory-miss penalties.

## General Analytical Model

For a program of N instructions:

Base cycles ≈ N + PipelineFillDrainOverhead

Expected dependency stall cycles ≈
    N × p_immediate_load_use × stall_cost

Expected memory miss cycles ≈
    N_memory_accesses × miss_rate × miss_penalty

Estimated CPI ≈
    (Base cycles + dependency stalls + memory miss cycles) / N

## Effect of Forwarding Coverage

Improving forwarding coverage primarily reduces stalls caused by ALU RAW dependencies. For programs containing many dependent arithmetic instructions, EX/MEM and MEM/WB bypass paths can keep CPI closer to one. However, an immediate load-use dependency remains sensitive to the time at which the loaded data becomes available.

## Effect of Cache Hit Rate

Improving cache hit rate primarily reduces the memory component of CPI. The benefit becomes increasingly important as the program contains more loads and stores or as the miss penalty increases. A pipeline with excellent forwarding can still suffer substantial CPI degradation when the MEM stage frequently waits for lower memory.

## Trade-Off Discussion

| Improvement | Main Benefit | Most Important When |
| --- | --- | --- |
| Better forwarding coverage | Reduces data-dependency stalls | Program contains many dependent ALU operations |
| Higher cache hit rate | Reduces memory stall penalty | Program contains many memory accesses or expensive misses |
| Both combined | Keeps execution and memory stalls low | Balanced workloads with dependencies and memory traffic |

## Result / Observation

The project demonstrates that forwarding and cache performance address different sources of pipeline inefficiency. Forwarding reduces dependency-related stalls, while a higher cache hit rate reduces memory-related stalls. The best overall CPI requires improvement in both areas.

# Deliverable 10: Complete End-to-End Execution Demonstration

## Purpose

The final demonstration executes the benchmark from instruction fetch through retirement while recording pipeline states and control events.

## End-to-End Execution Flow

STEP 1  Load six benchmark instructions
    |
STEP 2  Initialize 32-register register file and data memory
    |
STEP 3  Fetch I1 and then overlap subsequent instructions
    |
STEP 4  Detect I1 -> I2 immediate load-use dependency
    |
STEP 5  Freeze PC and IF/ID; inject one bubble
    |
STEP 6  Resume I2 and evaluate forwarding
    |
STEP 7  Forward dependent ALU results through EX/MEM or MEM/WB
    |
STEP 8  Access data memory for the load in MEM stage
    |
STEP 9  Write results back and retire instructions
    |
STEP 10 Compute CPI and analytical L1 miss metrics
    |
END    Report 11 actual simulation cycles and one stall

## Actual Execution Summary

| Metric | Observed / Computed Value |
| --- | --- |
| Benchmark instructions | 6 |
| Actual simulation cycles | 11 |
| Load-use stalls | 1 |
| Scenario B CPI | 1.83 |
| Analytical Scenario C CPI | 1.97 |
| Analytical speedup vs 20-cycle baseline | 1.69× |
| Forwarding modes observed | MEM/WB -> EX and EX/MEM -> EX |

## Result / Observation

The implementation completes the complete pipeline workflow and produces a hardware-style execution trace. The trace demonstrates the required load-use stall and both categories of forwarding behavior required by the assignment.

# Reflection: Design Decisions, SDG 9 Relevance and Learning Outcomes

## Design Decisions

The design separates three concerns: pipeline state movement, hazard detection, and forwarding selection. This separation makes the behavior easier to explain and aligns the implementation with the conceptual processor components taught in computer architecture. The C++ model provides executable cycle-by-cycle behavior, while the Verilog modules document the intended combinational hardware interfaces for hazard detection and forwarding.

The minimal one-cycle load-use stall was selected because forwarding alone cannot provide the loaded data early enough to the immediately dependent instruction. For the later ALU dependency chain, bypassing was selected instead of repeated stalls because the producer's result is available in later pipeline registers.

## Challenges and Learning

Understanding exactly when a value becomes available in a pipelined processor.

Distinguishing hazards that can be solved by forwarding from load-use hazards that require a stall.

Maintaining correct next-state behavior across multiple pipeline registers.

Coordinating PC freezing, IF/ID freezing, and bubble injection.

Connecting cycle-level simulation results to CPI, speedup, and memory hierarchy performance.

## SDG 9 – Industry, Innovation and Infrastructure

This project relates to SDG 9 because efficient processor and memory-hierarchy design supports reliable and innovative digital infrastructure. Reducing unnecessary pipeline stalls improves computational efficiency, while careful memory-hierarchy design reduces latency-related performance losses. The project demonstrates how low-level architectural decisions can contribute to more capable computing systems and stronger technology infrastructure.

## Result / Observation

The project provided practical experience in translating theoretical pipeline concepts into executable logic and interpreting the resulting cycle traces and performance metrics.

# Overall Conclusion

The submitted CSA1205 implementation successfully demonstrates the central concepts required for pipeline performance analysis. A five-stage RISC pipeline is modeled with IF, ID, EX, MEM, and WB stages and explicit inter-stage pipeline registers. The benchmark includes a sequence of dependent instructions that exposes both an immediate load-use hazard and a chain of ALU RAW dependencies.

The load-use hazard is detected by comparing the destination register of a memory-reading instruction in ID/EX with the source registers of the instruction in IF/ID. When the condition is true, the implementation freezes the PC and IF/ID register and injects a bubble. The benchmark produces one unavoidable stall.

The forwarding logic compares ID/EX source registers with EX/MEM and MEM/WB destination registers and selects the newest available result. The execution trace demonstrates one MEM/WB-to-EX forwarding event and multiple EX/MEM-to-EX forwarding events, allowing the ALU dependency chain to proceed without additional stalls.

The C++ simulator completes the six-instruction benchmark in 11 cycles, producing a CPI of 1.83 for the forwarding configuration. Using the submitted analytical cache model of a 10% L1 miss rate and an 8-cycle miss penalty, the expected execution time becomes 11.8 cycles and the expected CPI becomes approximately 1.97. Relative to the 20-cycle stall-only reference used by the implementation, the expected speedup is 1.69×.

Overall, the project shows that pipeline forwarding and memory hierarchy performance must be considered together. Forwarding reduces data-dependency stalls, while cache misses can introduce a separate source of pipeline delay. The combined analysis therefore provides a practical demonstration of how processor datapath control and memory-system behavior jointly determine CPI and throughput.

# References

CSA1205 – Computer Architecture Assignment Specification and Assessment Rubrics.

Submitted project source: pipeline_sim.cpp – hardware-oriented five-stage pipeline simulator.

Submitted project source: forwarding_unit.v – EX/MEM and MEM/WB forwarding logic.

Submitted project source: hazard_unit.v – load-use hazard detection interface.

Submitted project source: app.py – Flask-based simulation backend.

Submitted project README – project overview and implementation features.