SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1-CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1-Constraint-Based Problem Solving |
| Weightage: | 60% | Total Marks: | 25 |
| CO5Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | R.MOHANRAM | Reg.No.: | 192521117 |
| Department: | B.Tech IT | Date: | 3-9-26 |

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

Problem Understanding

The real-time embedded system contains multiple sensors that generate interrupts every 1 ms. The system must process these sensor events with very low latency while keeping CPU utilization below 15%. The system must also use vectored interrupt handling and select either memory-mapped I/O or I/O-mapped I/O for optimal performance.

Therefore, the proposed mechanism must provide:

- Fast interrupt response.

- Low CPU overhead.

- Support for multiple sensors.

- Deterministic real-time operation.

- CPU utilization below 15%.

- Vectored interrupt handling.

- Low-latency communication between the CPU and I/O devices.

2. Constraint Analysis

| Constraint | Requirement | Design Decision |
| --- | --- | --- |
| CPU utilization | Below 15% | Keep ISR short and defer lengthy processing |
| Sensor interrupts | Every 1 ms | Use a fast interrupt controller |
| Multiple sensors | Concurrent interrupt requests | Assign separate interrupt vectors |
| Interrupt mechanism | Vectored interrupt handling | Directly jump to the appropriate ISR |
| I/O technique | Select for minimum latency | Use Memory-Mapped I/O |
| Real-time operation | Predictable response | Use priority-based interrupt handling |

The major challenge is that frequent interrupts can consume significant CPU time. If each sensor interrupt requires extensive processing, the CPU utilization may exceed the required 15% limit.

Therefore, the Interrupt Service Routine (ISR) should perform only essential operations such as reading sensor data, clearing the interrupt flag, and placing data into a buffer.

3. Proposed Solution

The proposed I/O communication mechanism consists of:

- Multiple Sensors

- Vectored Interrupt Controller

- Priority-Based Interrupt Handling

- Memory-Mapped I/O

- Short Interrupt Service Routines

- RAM Buffers for Sensor Data

- Deferred Background Processing

Architecture Diagram

┌─────────────────┐

│    Sensor 1     │

└────────┬────────┘

│ Interrupt

┌────────▼────────┐

│    Sensor 2     │

└────────┬────────┘

│ Interrupt

▼

┌─────────────────────────┐

│ Vectored Interrupt      │

│ Controller              │

│                         │

│ Vector 1 → ISR Sensor 1 │

│ Vector 2 → ISR Sensor 2 │

│ Vector 3 → ISR Sensor 3 │

│ Vector N → ISR Sensor N │

└────────────┬────────────┘

│

▼

┌─────────────────┐

│       CPU       │

│                 │

│ Short ISR       │

│ Read Data       │

│ Clear Interrupt │

└────────┬────────┘

│

▼

┌─────────────────────────┐

│   Memory-Mapped I/O     │

│                         │

│ Sensor Control Registers│

│ Sensor Data Registers   │

└────────────┬────────────┘

│

▼

┌─────────────────┐

│   RAM Buffer    │

└────────┬────────┘

│

▼

┌─────────────────┐

│ Background Task │

│ Data Processing │

└─────────────────┘

4. Working Mechanism

Step 1: Sensor Generates an Interrupt

Each sensor generates an interrupt every 1 ms when new data is available.

The interrupt request is sent to the Vectored Interrupt Controller.

Step 2: Interrupt Controller Identifies the Source

Each sensor is assigned a unique interrupt vector.

For example:

| Sensor | Interrupt Vector |
| --- | --- |
| Sensor 1 | Vector 10 |
| Sensor 2 | Vector 11 |
| Sensor 3 | Vector 12 |
| Sensor 4 | Vector 13 |

When an interrupt occurs, the controller directly identifies the corresponding Interrupt Service Routine.

This eliminates the need for the CPU to repeatedly check every device.

Step 3: CPU Executes the Appropriate ISR

The CPU directly jumps to the required ISR.

Example:

Sensor 2 Interrupt

↓

Interrupt Controller

↓

Vector 11

↓

Sensor 2 ISR

The ISR performs only three operations:

- Read sensor data.

- Store data in a RAM buffer.

- Clear the interrupt request.

The ISR must avoid complex calculations.

Step 4: Memory-Mapped I/O Access

The proposed design uses Memory-Mapped I/O (MMIO).

In Memory-Mapped I/O, sensor registers are assigned addresses within the processor's normal memory address space.

Example:

Sensor 1 Data Register    → 0x40000000

Sensor 2 Data Register    → 0x40000004

Sensor 3 Data Register    → 0x40000008

Sensor Control Register   → 0x4000000C

The CPU accesses these registers using normal load and store instructions.

LOAD R1, [0x40000000]

STORE R1, [RAM_BUFFER]

This provides faster and simpler communication compared with using separate I/O instructions.

5. Memory-Mapped I/O vs I/O-Mapped I/O

Memory-Mapped I/O

In memory-mapped I/O, I/O devices share the same address space as memory.

Advantages:

- Uses normal CPU instructions.

- Fast register access.

- Simple programming model.

- Suitable for modern embedded processors.

- Supports efficient compiler optimization.

- Better integration with cache-controlled and high-performance architectures.

I/O-Mapped I/O

In I/O-mapped I/O, devices use a separate address space.

Disadvantages for this system:

- Requires special I/O instructions.

- Can increase instruction handling complexity.

- Less suitable for high-speed embedded sensor interfaces.

Selected Technique

Memory-Mapped I/O is selected because the system requires minimum latency and frequent access to sensor registers.

6. CPU Utilization Analysis

The CPU utilization due to interrupt processing can be estimated using:

CPU Utilization=Total Interrupt Processing TimeTotal Available CPU Time×100

Assume:

- Each sensor interrupt occurs every 1 ms.

- The ISR is designed to complete in approximately 10 µs.

- Four sensors generate interrupts.

Total interrupt processing time per millisecond:

4×10 μs=40 μs

CPU utilization:

CPU Utilization=401000×100
=4%

Therefore:

CPU utilization due to interrupts≈4%

This is below the required constraint of:

15%

The remaining CPU capacity can be used for background processing and other real-time tasks.

7. Application of Concepts

The following Computer Architecture concepts are applied:

Vectored Interrupts

Each interrupt source has a unique vector. The CPU directly accesses the correct ISR, reducing interrupt latency.

Priority-Based Interrupt Handling

Critical sensors can be assigned higher priority. If multiple interrupts occur simultaneously, the interrupt controller services the highest-priority request first.

Memory-Mapped I/O

Sensor registers are mapped into the CPU address space, allowing fast access through standard instructions.

Interrupt Service Routine Optimization

The ISR performs only essential operations, reducing CPU overhead.

Buffer-Based Processing

Sensor data is quickly transferred into RAM, while complex processing is performed later by a background task.

8. Final Proposed Mechanism

The final I/O communication mechanism is:

Sensors

↓

Vectored Interrupt Controller

↓

Priority Selection

↓

CPU

↓

Short Interrupt Service Routine

↓

Memory-Mapped I/O Register Access

↓

RAM Buffer

↓

Deferred Background Processing

9. Justification

The proposed architecture satisfies all the specified constraints.

- CPU utilization remains below 15% because ISRs are kept short.

- Multiple sensors are supported through separate interrupt vectors.

- 1 ms interrupt generation is handled efficiently using vectored interrupts.

- Interrupt latency is reduced because the CPU directly jumps to the appropriate ISR.

- Memory-Mapped I/O provides low-latency device access using normal processor instructions.

- Background processing prevents lengthy calculations inside the ISR, ensuring that frequent interrupts do not overload the CPU.

10. Conclusion

A real-time embedded I/O communication mechanism was constructed using Memory-Mapped I/O and vectored interrupt handling. Each sensor is assigned a separate interrupt vector, allowing the CPU to immediately execute the corresponding ISR. The ISR performs only essential operations and stores sensor data in RAM for later processing.

The estimated interrupt CPU utilization is approximately 4%, which satisfies the requirement of keeping CPU utilization below 15%. Therefore, the proposed architecture provides a low-latency, efficient, scalable, and real-time solution for multiple sensors generating interrupts every 1 ms.

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

Problem Understanding

The system requires a high-speed storage I/O subsystem with the following constraints:

- Storage throughput must be greater than 3 GB/s.

- The system must use NVMe over PCIe.

- The CPU should not handle bulk data transfer.

- A DMA controller must transfer data.

- The CPU must receive an interrupt notification after the transfer.

Therefore, the proposed architecture must provide high-speed communication between the NVMe SSD, main memory, and CPU while minimizing CPU involvement.

2. Constraint Analysis

| Constraint | Requirement | Design Decision |
| --- | --- | --- |
| Storage speed | > 3 GB/s | Use NVMe SSD |
| Communication interface | NVMe over PCIe | Use high-bandwidth PCIe connection |
| CPU involvement | No bulk data transfer | Use DMA |
| Data transfer | Direct device-to-memory | DMA transfer |
| Transfer completion | CPU notification required | Interrupt notification |
| Performance | Low latency | NVMe command queues and PCIe |

The most important requirement is to avoid transferring every byte through the CPU. If the CPU performs bulk data movement, CPU utilization increases and the system cannot efficiently maintain high throughput.

Therefore, DMA is selected for bulk data transfer.

3. Proposed I/O Subsystem Architecture

┌──────────────────────┐

│         CPU          │

│                      │

│ NVMe Command Setup   │

└──────────┬───────────┘

│

│ PCIe Commands

▼

┌─────────────────────────────┐

│       PCIe Interface        │

│                             │

└──────────────┬──────────────┘

│

▼

┌─────────────────────────────┐

│       NVMe Controller       │

│                             │

│ Submission Queue            │

│ Completion Queue            │

└──────────────┬──────────────┘

│

▼

┌────────────────┐

│    NVMe SSD    │

└────────┬───────┘

│

Bulk Data Transfer

│

▼

┌────────────────┐

│ DMA Controller │

└────────┬───────┘

│

Direct Memory Access

│

▼

┌────────────────┐

│   Main Memory  │

│      (RAM)     │

└────────┬───────┘

│

Completion Interrupt

│

▼

CPU

4. Working of the Proposed System

The operation of the I/O subsystem takes place in the following stages.

Step 1: CPU Creates an NVMe Command

The CPU does not transfer the actual bulk data.

Instead, it creates an NVMe command containing information such as:

- Read or write operation.

- Storage block address.

- Number of blocks.

- Main memory buffer address.

Example:

Operation        : READ

Storage Address  : Block 5000

Data Size        : 4 MB

RAM Address      : 0x80000000

The CPU places this command into the NVMe Submission Queue.

Step 2: Command Travels Through PCIe

The NVMe controller receives the command through the PCIe interface.

PCIe is selected because it provides:

- High bandwidth.

- Low latency.

- Point-to-point communication.

- Scalable performance.

A PCIe connection provides sufficient bandwidth to support the required throughput of more than 3 GB/s.

Step 3: NVMe Controller Accesses the SSD

The NVMe controller processes the command and accesses the requested data in the NVMe SSD.

NVMe is specifically designed for high-performance solid-state storage and supports multiple command queues.

This allows multiple storage operations to be processed efficiently.

Step 4: DMA Transfers Bulk Data Directly to RAM

After accessing the data, the bulk data is transferred directly to main memory using Direct Memory Access (DMA).

The data path is:

NVMe SSD

↓

NVMe Controller

↓

PCIe

↓

DMA Engine

↓

Main Memory

The CPU does not need to execute individual instructions for each byte or block of data.

This is the most important feature for satisfying the CPU utilization constraint.

5. DMA Transfer Mechanism

The DMA controller is configured with:

- Source address.

- Destination address.

- Transfer size.

- Transfer direction.

For example:

Source Address      → NVMe Data Buffer

Destination Address → RAM Buffer

Transfer Size       → 4 MB

Direction           → Device to Memory

Once configured, DMA performs the complete transfer independently.

DMA Operation

CPU Configures DMA

↓

DMA Receives Transfer Request

↓

DMA Requests System Bus

↓

Bus Arbitration

↓

DMA Transfers Data

↓

RAM Receives Data

↓

Transfer Completed

The CPU is free to execute other tasks while DMA performs the data transfer.

6. Interrupt Notification

After the DMA transfer is completed, the system generates an interrupt.

The sequence is:

DMA Transfer Complete

↓

DMA / NVMe Controller

↓

Interrupt Request

↓

Interrupt Controller

↓

CPU Interrupt Service Routine

↓

Completion Queue Checked

↓

Application Notified

The CPU interrupt handler performs only essential tasks:

- Identify the completed transfer.

- Check the NVMe completion queue.

- Mark the I/O operation as complete.

- Notify the operating system or application.

This avoids continuous polling by the CPU.

7. Data Transfer Path

Read Operation

Application

↓

CPU Creates NVMe Command

↓

NVMe Submission Queue

↓

PCIe

↓

NVMe SSD

↓

DMA Transfer

↓

Main Memory

↓

Completion Interrupt

↓

CPU / Application

Write Operation

Application

↓

Data Available in Main Memory

↓

DMA Transfer

↓

PCIe

↓

NVMe Controller

↓

NVMe SSD

↓

Completion Interrupt

↓

CPU

8. Throughput Analysis

The required throughput is:

Required Throughput>3 GB/s

Assume the PCIe and NVMe subsystem provides an effective transfer rate of 4 GB/s.

Then:

4 GB/s>3 GB/s

Therefore, the proposed system satisfies the throughput requirement.

For a 4 GB data transfer:

Transfer Time=Data SizeTransfer Rate
=4 GB4 GB/s
=1 second

Thus, the DMA-based NVMe system can efficiently perform high-speed bulk transfers without requiring continuous CPU involvement.

9. CPU-Based Transfer vs DMA Transfer

| Feature | CPU-Based Transfer | DMA-Based Transfer |
| --- | --- | --- |
| Bulk data movement | CPU transfers data | DMA transfers data |
| CPU utilization | High | Low |
| Transfer efficiency | Lower for large data | High |
| CPU availability | CPU remains busy | CPU can perform other tasks |
| Suitable for NVMe | Less efficient | Highly suitable |
| Interrupt usage | Frequent CPU involvement | Completion interrupt only |
| Throughput | Limited by CPU overhead | High |

Therefore, DMA is the optimal choice for this high-speed storage system.

10. Application of Computer Architecture Concepts

The proposed solution applies the following concepts:

NVMe

NVMe provides a high-performance storage protocol designed for modern solid-state drives.

PCIe

PCIe provides a high-speed communication interface between the processor and NVMe storage device.

DMA

DMA allows direct transfer of bulk data between the storage device and main memory without continuous CPU involvement.

Interrupt-Driven Completion

The CPU receives an interrupt only after the transfer is completed.

NVMe Queues

Submission and completion queues allow efficient management of multiple I/O commands.

11. Final Proposed Architecture

CPU

│

│ Creates NVMe Command

▼

NVMe Submission Queue

│

▼

PCIe Interface

│

▼

NVMe Controller

│

▼

NVMe SSD

│

High-Speed Data

│

▼

DMA Engine

│

Direct Transfer

│

▼

Main Memory

│

▼

Completion Interrupt

│

▼

CPU

12. Justification

The proposed architecture satisfies all constraints because:

- NVMe over PCIe provides the required high-speed storage interface.

- The system can achieve more than 3 GB/s throughput with an appropriate PCIe/NVMe configuration.

- DMA handles bulk data transfer, preventing excessive CPU utilization.

- The CPU only performs command setup and completion handling.

- Interrupt notification informs the CPU when the transfer is completed.

- NVMe queues improve performance and allow efficient handling of multiple storage operations.

- PCIe provides a scalable, high-bandwidth connection suitable for high-performance storage.

13. Conclusion

A high-speed I/O subsystem was designed using NVMe over PCIe with DMA-based data transfer. The CPU creates and submits NVMe commands, while the DMA engine transfers bulk data directly between the NVMe storage subsystem and main memory.

After completion, an interrupt notifies the CPU, eliminating the need for continuous polling or CPU-based bulk transfer. The proposed architecture satisfies the required throughput of more than 3 GB/s, minimizes CPU involvement, and provides an efficient and scalable solution for high-performance storage communication.

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

Problem Understanding

The system must support multiple peripheral devices with the following constraints:

- 6 peripherals using USB

- 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use both hardware and software interrupts

- Ensure fair bus arbitration

The objective is to construct a communication architecture that allows all eight devices to communicate efficiently with the CPU without allowing one device or group of devices to monopolize the processor or communication bus.

2. Constraint Analysis

| Constraint | Requirement | Proposed Solution |
| --- | --- | --- |
| USB devices | 6 peripherals | Use USB Host Controller and USB Hub |
| High-speed devices | 2 devices | Use Thunderbolt Controller |
| Interrupt starvation | Must be avoided | Priority with aging / fair interrupt scheduling |
| Hardware interrupts | Required | Use device-generated hardware interrupts |
| Software interrupts | Required | Use deferred software interrupt handling |
| Bus fairness | Required | Use round-robin arbitration |
| Multiple devices | 8 total devices | Centralized interrupt and bus management |

The main challenge is balancing high-speed Thunderbolt traffic with multiple USB peripherals while ensuring that low-priority devices are not permanently delayed.

3. Proposed Multi-Device Communication Architecture

┌───────────────────┐

│       CPU         │

└─────────┬─────────┘

│

┌───────────────┼────────────────┐

│               │                │

▼               ▼                ▼

┌────────────────┐ ┌─────────────┐ ┌────────────────┐

│ Interrupt      │ │ Software    │ │ Bus Arbitration│

│ Controller     │ │ Interrupt   │ │ Controller     │

└────────┬───────┘ └─────────────┘ └────────┬───────┘

│                                  │

Hardware Interrupts                       │ Fair Arbitration

│                                  │

┌────────┴────────┐                 ┌───────┴────────┐

│                 │                 │                │

▼                 ▼                 ▼                ▼

┌──────────────┐ ┌──────────────┐ ┌────────────┐ ┌──────────────┐

│ USB Host     │ │ Thunderbolt  │ │ USB Bus    │ │ Thunderbolt  │

│ Controller   │ │ Controller   │ │            │ │ Bus          │

└──────┬───────┘ └──────┬───────┘ └────────────┘ └──────────────┘

│                │

│                │

┌─────┴──────┐   ┌─────┴──────┐

│ USB Hub    │   │ High-Speed │

└─────┬──────┘   │ Devices    │

│          └────────────┘

│             Device 7

│             Device 8

│

┌─────┼─────┬─────┬─────┬─────┐

▼     ▼     ▼     ▼     ▼     ▼

USB1  USB2  USB3  USB4  USB5  USB6

4. USB Peripheral Communication

The six peripheral devices are connected through a USB Host Controller, which manages communication between the CPU and USB devices.

The architecture is:

CPU

│

▼

USB Host Controller

│

▼

USB Hub

│

├── USB Device 1

├── USB Device 2

├── USB Device 3

├── USB Device 4

├── USB Device 5

└── USB Device 6

Examples of USB peripherals may include:

- Keyboard

- Mouse

- Printer

- Scanner

- Sensor

- External USB device

The USB Host Controller manages communication and generates hardware interrupts when important events occur.

5. Thunderbolt High-Speed Communication

The two high-speed devices are connected using a Thunderbolt controller.

CPU

│

▼

Thunderbolt Controller

│

├── High-Speed Device 1

│

└── High-Speed Device 2

Thunderbolt is suitable for devices requiring high bandwidth, such as:

- External high-speed storage

- High-resolution display devices

- Professional data acquisition devices

The Thunderbolt controller handles high-speed data communication and sends hardware interrupts when a significant event or transfer completion occurs.

6. Hardware Interrupt Mechanism

Hardware interrupts are generated directly by physical devices.

For example:

USB Device

│

│ Hardware Interrupt Request

▼

Interrupt Controller

│

▼

CPU

│

▼

Interrupt Service Routine

Each major device controller is assigned an interrupt vector.

Example:

| Device Controller | Interrupt Type | Priority |
| --- | --- | --- |
| Thunderbolt Device 1 | Hardware Interrupt | High |
| Thunderbolt Device 2 | Hardware Interrupt | High |
| USB Controller | Hardware Interrupt | Medium |
| USB Device Events | Hardware Interrupt | Medium |
| Background Services | Software Interrupt | Low |

Hardware interrupts are used for events that require immediate attention.

7. Software Interrupt Mechanism

Software interrupts are used for tasks that do not require immediate hardware-level processing.

The sequence is:

Hardware Interrupt

↓

Short Hardware ISR

↓

Store Event Information

↓

Trigger Software Interrupt

↓

Deferred Processing

For example, when a USB device generates a hardware interrupt, the hardware ISR performs only critical operations.

The remaining processing is transferred to a software interrupt or deferred service routine.

This reduces interrupt processing time and improves CPU availability.

8. Preventing Interrupt Starvation

Interrupt starvation occurs when low-priority interrupts are continuously delayed because high-priority interrupts keep arriving.

To prevent this problem, the proposed system uses priority scheduling with aging.

Priority with Aging

Initially, high-speed and time-critical devices receive higher priority.

However, if a low-priority device waits for a long time, its priority gradually increases.

The effective priority can be represented as:

Effective Priority=Base Priority+Waiting Time×Aging Factor

For example:

| Device | Base Priority | Waiting Time | Effective Priority |
| --- | --- | --- | --- |
| Thunderbolt 1 | 10 | 1 | 10.5 |
| Thunderbolt 2 | 10 | 1 | 10.5 |
| USB Device 1 | 5 | 8 | 9 |
| USB Device 2 | 4 | 15 | 11.5 |

In this example, a lower-priority USB device that has waited for a long period receives increased priority.

Therefore, no device remains permanently blocked.

9. Fair Bus Arbitration

Multiple devices may request access to the communication bus simultaneously.

A Round-Robin Bus Arbitration mechanism is used.

Working Principle

Each device receives a turn for bus access.

USB1 → USB2 → USB3 → USB4 → USB5 → USB6

↓

TB Device 1

↓

TB Device 2

↓

Repeat Cycle

If a device does not require the bus during its turn, the arbiter immediately moves to the next requesting device.

This prevents one device from continuously controlling the bus.

10. Bus Arbitration Process

Multiple Bus Requests

│

▼

Bus Arbiter

│

▼

Check Request Queue

│

▼

Round-Robin Selection

│

▼

Grant Bus Access

│

▼

Data Transfer

│

▼

Release Bus

│

▼

Next Device

This method ensures fair access among USB and Thunderbolt devices.

11. Complete Communication Flow

The complete operation of the system is:

USB / Thunderbolt Device

│

▼

Hardware Event

│

▼

Hardware Interrupt

│

▼

Interrupt Controller

│

▼

CPU Executes Short ISR

│

▼

Event Stored in Buffer

│

▼

Software Interrupt Generated

│

▼

Deferred Processing

│

▼

Bus Request

│

▼

Round-Robin Arbitration

│

▼

Device Receives Bus Access

│

▼

Data Transfer Completed

12. Application of Computer Architecture Concepts

The proposed architecture applies the following concepts:

USB Interface

USB is used to support six general-purpose peripheral devices through a host controller and hub.

Thunderbolt Interface

Thunderbolt is used for two high-speed devices requiring high bandwidth and low latency.

Hardware Interrupts

Hardware interrupts immediately notify the CPU about important device events.

Software Interrupts

Software interrupts are used for deferred or background processing.

Interrupt Priority and Aging

Priority handling ensures fast service for critical devices, while aging prevents starvation.

Round-Robin Arbitration

Round-robin arbitration ensures that every requesting device receives a fair opportunity to access the bus.

13. Constraint Satisfaction

| Given Constraint | How the Solution Satisfies It |
| --- | --- |
| 6 USB peripherals | USB Host Controller and Hub support six devices |
| 2 Thunderbolt devices | Dedicated Thunderbolt Controller manages both devices |
| Avoid interrupt starvation | Priority scheduling with aging |
| Hardware interrupts | Generated by USB and Thunderbolt controllers |
| Software interrupts | Used for deferred/background processing |
| Fair bus access | Round-robin arbitration |

14. Justification

The proposed architecture is suitable because it separates device communication according to performance requirements.

USB provides an efficient interface for six standard peripherals, while Thunderbolt supports two high-speed devices requiring greater bandwidth.

Hardware interrupts provide immediate notification for critical events. However, processing every event completely inside a hardware ISR could increase CPU overhead. Therefore, lengthy tasks are deferred using software interrupts.

Priority scheduling with aging ensures that high-priority devices receive fast service while preventing low-priority devices from suffering interrupt starvation.

Finally, round-robin bus arbitration ensures fair communication access among all requesting devices.

15. Conclusion

A multi-device communication architecture was constructed to support six USB peripherals and two high-speed Thunderbolt devices. The system uses USB and Thunderbolt controllers for device communication, hardware interrupts for immediate event notification, and software interrupts for deferred processing.

Priority scheduling with aging prevents interrupt starvation, while round-robin bus arbitration ensures fair access to communication resources. Therefore, the proposed architecture satisfies all specified constraints and provides an efficient, scalable, and fair multi-device I/O communication system.

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

Problem Understanding

A data acquisition system must continuously receive and store data from a high-speed device. The system has the following constraints:

- Continuous data stream of 500 MB/s

- Minimal CPU intervention is required

- Comparison between DMA transfer and interrupt-driven I/O is required

- Memory-Mapped I/O (MMIO) must be used for device registers

The main objective is to design an efficient data acquisition system capable of handling 500 MB/s continuously without overloading the CPU.

2. Constraint Analysis

| Constraint | Requirement | Design Decision |
| --- | --- | --- |
| Data rate | 500 MB/s continuous | High-speed DMA transfer |
| CPU involvement | Minimal | CPU only configures DMA |
| Transfer method | Compare DMA and interrupts | Select DMA for bulk transfer |
| Device registers | Memory-Mapped I/O | Map registers into CPU address space |
| Continuous operation | No data loss | Use circular/ring buffer |
| Completion handling | Efficient notification | Interrupt after DMA block completion |

The most important challenge is that the incoming data stream is continuous. If the CPU processes each data item using interrupts, the number of interrupts can become extremely large.

Therefore, DMA with buffered block transfers is the most suitable solution.

3. Proposed Data Acquisition Architecture

┌───────────────────┐

│       CPU         │

│                   │

│ DMA Configuration │

└─────────┬─────────┘

│

│ MMIO Access

▼

┌───────────────────┐

│ Device Registers  │

│   Memory-Mapped   │

│       I/O         │

└─────────┬─────────┘

│

▼

┌───────────────────┐

│ Data Acquisition  │

│      Device       │

└─────────┬─────────┘

│

Continuous Data Stream

500 MB/s

│

▼

┌───────────────────┐

│   DMA Controller  │

└─────────┬─────────┘

│

Direct Transfer

│

▼

┌───────────────────┐

│ Circular Buffer   │

│    Main Memory    │

└─────────┬─────────┘

│

Block Completion

Interrupt

│

▼

CPU

│

▼

Data Processing Task

4. Proposed Working Mechanism

The system operates using the following steps.

Step 1: Configure Device Registers Using Memory-Mapped I/O

The data acquisition device contains control and status registers.

These registers are mapped into the CPU memory address space.

For example:

Device Control Register  → 0x50000000

Device Status Register   → 0x50000004

DMA Source Register      → 0x50000008

DMA Destination Register → 0x5000000C

The CPU accesses these registers using normal memory instructions.

Example:

WRITE [0x50000000] → START_ACQUISITION

READ  [0x50000004] → DEVICE_STATUS

Memory-mapped I/O is useful because it provides a simple and efficient method for configuring and monitoring the acquisition device.

5. Step 2: Configure DMA Controller

The CPU configures the DMA controller before data acquisition begins.

The DMA configuration contains:

- Source address

- Destination memory address

- Transfer size

- Transfer mode

- Interrupt enable

Example:

Source       → Data Acquisition Device

Destination  → RAM Circular Buffer

Block Size   → 4 MB

Mode         → Continuous / Circular DMA

Interrupt    → Enabled After Block Completion

After configuration, the CPU starts the acquisition process.

The CPU does not transfer the 500 MB/s data manually.

6. Step 3: Continuous DMA Transfer

The data path is:

Data Acquisition Device

│

│ 500 MB/s

▼

DMA Controller

│

│ Direct Memory Transfer

▼

Main Memory

│

▼

Circular Buffer

The DMA controller transfers data directly into RAM.

A circular buffer is used because the incoming data is continuous.

Circular Buffer Operation

┌───────────────────────────────┐

│        Main Memory            │

│                               │

│  Buffer A → Buffer B → Buffer C

│       ↑                       │

│       └──── Continuous ───────┘

│            DMA Transfer       │

└───────────────────────────────┘

When one buffer becomes full, DMA automatically moves to the next buffer.

The CPU or processing task can process previously completed buffers.

This mechanism supports continuous acquisition with reduced data loss.

7. DMA-Based Data Acquisition

The operation is:

CPU Configures DMA

│

▼

Device Starts Generating Data

│

▼

DMA Receives Data

│

▼

DMA Transfers Data Directly

│

▼

RAM Buffer Filled

│

▼

Block Completion Interrupt

│

▼

CPU Processes Completed Block

The CPU is involved mainly in:

- Initial configuration.

- Handling block completion.

- Starting the next processing task.

Therefore, CPU intervention remains minimal.

8. Interrupt-Driven I/O Method

In interrupt-driven I/O, the device interrupts the CPU whenever data is available.

The process is:

Device Generates Data

│

▼

Hardware Interrupt

│

▼

CPU Executes ISR

│

▼

CPU Reads Device Data

│

▼

CPU Stores Data in RAM

│

▼

Next Data Interrupt

At a continuous data rate of 500 MB/s, frequent interrupts would cause a large amount of CPU overhead.

The CPU would repeatedly:

- Stop its current task.

- Save processor context.

- Execute the ISR.

- Read the device.

- Transfer data.

- Restore the previous context.

This makes interrupt-driven I/O inefficient for continuous high-speed data transfer.

9. DMA vs Interrupt-Driven I/O Comparison

| Feature | DMA Transfer | Interrupt-Driven I/O |
| --- | --- | --- |
| Data movement | DMA controller | CPU |
| CPU involvement | Very low | High |
| Suitable for 500 MB/s | Yes | Less suitable |
| Continuous transfer | Efficient | High overhead |
| Number of interrupts | Per block | Frequent/per data event |
| CPU availability | High | Reduced |
| Transfer efficiency | High | Lower |
| Best use | Bulk continuous data | Small or infrequent data |

Therefore, DMA is selected as the primary transfer mechanism.

10. Data Rate Analysis

The system must handle:

500 MB/s

Assume the DMA transfers data in 4 MB blocks.

The number of DMA blocks transferred per second is:

Blocks per second=500 MB/s4 MB
=125 blocks/second

Therefore, the system requires approximately:

125 DMA completion events per second

If an interrupt is generated after every 4 MB block, the CPU handles only approximately 125 completion interrupts per second.

This is significantly more efficient than generating interrupts for small individual data units.

11. CPU Utilization Comparison

DMA-Based Transfer

With DMA:

CPU → Configure DMA

DMA → Transfer 500 MB/s

CPU → Receive Block Completion Interrupt

The CPU does not participate in the actual movement of every byte.

Therefore:

CPU utilization is low

Interrupt-Driven Transfer

With interrupt-driven I/O:

Device → Interrupt CPU

CPU → Read Data

CPU → Store Data

Device → Interrupt CPU Again

CPU → Repeat

At high data rates, CPU utilization increases significantly.

Therefore:

Interrupt-driven I/O has higher CPU overhead

12. Application of Computer Architecture Concepts

The proposed solution applies the following concepts.

Memory-Mapped I/O

The device control and status registers are mapped into the processor's address space.

DMA

DMA transfers data directly from the acquisition device to RAM.

Interrupt Handling

The CPU receives an interrupt only after a DMA block is completed.

Circular Buffering

A circular buffer supports continuous streaming and prevents the acquisition process from stopping between transfers.

Bus Arbitration

The DMA controller obtains bus access to transfer data to main memory without requiring the CPU to execute individual data transfer instructions.

13. Final Proposed System

┌─────────────────┐

│       CPU       │

│                 │

│ Configure Device│

│ Configure DMA   │

└────────┬────────┘

│

Memory-Mapped

I/O

│

▼

┌─────────────────┐

│ Acquisition     │

│ Device          │

└────────┬────────┘

│

500 MB/s Data

│

▼

┌─────────────────┐

│ DMA Controller  │

└────────┬────────┘

│

Direct Transfer

│

▼

┌─────────────────┐

│ Circular Buffer │

│      RAM        │

└────────┬────────┘

│

Block Completed

│

▼

Interrupt

│

▼

CPU

14. Constraint Satisfaction

| Given Constraint | Solution |
| --- | --- |
| Continuous stream at 500 MB/s | High-speed DMA transfer |
| Minimal CPU intervention | CPU only configures and monitors DMA |
| Compare DMA vs interrupt I/O | DMA selected due to lower CPU overhead |
| Memory-mapped I/O | Used for device control and status registers |
| Continuous acquisition | Circular/ring buffer |
| Efficient notification | Interrupt after block completion |

15. Justification

DMA is the best solution for a continuous 500 MB/s data stream because transferring such a large amount of data through interrupt-driven CPU operations would create excessive CPU overhead.

Using memory-mapped I/O, the CPU can quickly configure and monitor the acquisition device through normal memory instructions.

The DMA controller transfers bulk data directly to RAM, allowing the CPU to perform other operations. Instead of generating interrupts for every small data unit, the system generates an interrupt after completing a large buffer block.

For a 4 MB buffer size, approximately 125 block completion events per second are required, which is much more manageable than frequent small-data interrupts.

The circular buffer further supports continuous operation by allowing DMA and CPU processing to occur simultaneously.

16. Conclusion

A high-speed data acquisition system was designed to handle a continuous 500 MB/s data stream using DMA and Memory-Mapped I/O.

The CPU uses memory-mapped registers to configure the acquisition device and DMA controller. The DMA controller transfers bulk data directly into a circular RAM buffer, requiring minimal CPU intervention. The CPU is notified only after block completion through interrupts.

Compared with interrupt-driven I/O, the DMA-based approach provides lower CPU utilization, higher throughput, fewer interrupts, and better support for continuous high-speed data acquisition.

Therefore, the proposed DMA-based architecture is the optimal solution for satisfying all the given constraints.

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Problem Understanding

The system requires an interrupt handling mechanism with the following constraints:

- Support nested interrupts

- Maximum interrupt latency must be less than 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

The objective is to design a fast and efficient interrupt system in which critical devices receive immediate CPU attention while lower-priority background tasks do not affect real-time interrupt performance.

2. Constraint Analysis

| Constraint | Requirement | Design Decision |
| --- | --- | --- |
| Nested interrupts | Required | Priority-based nested interrupt controller |
| Interrupt latency | < 10 µs | Fast vectored interrupt mechanism |
| High-priority devices | Immediate response | Dedicated interrupt vectors |
| Background tasks | Lower priority | Software interrupts |
| Multiple interrupt levels | Required | Interrupt priority levels |
| Critical events | Must not wait unnecessarily | Allow higher-priority interrupt preemption |

The major challenge is ensuring that a critical interrupt can interrupt a lower-priority ISR while keeping the total interrupt latency below 10 µs.

3. Proposed Interrupt Handling Architecture

┌────────────────────┐

│        CPU         │

└──────────┬─────────┘

│

▼

┌────────────────────┐

│ Interrupt Controller│

│                    │

│ Priority Manager   │

│ Vector Table       │

│ Nesting Support    │

└──────────┬─────────┘

│

┌────────────────────┼────────────────────┐

│                    │                    │

▼                    ▼                    ▼

┌────────────────┐   ┌────────────────┐   ┌────────────────┐

│ High Priority  │   │ Medium Priority│   │ Low Priority   │

│ Devices        │   │ Devices        │   │ Devices        │

│ Vectored IRQ   │   │ Hardware IRQ   │   │ Software Tasks │

└────────────────┘   └────────────────┘   └────────────────┘

│

▼

┌────────────────┐

│ Dedicated ISR  │

└────────────────┘

The interrupt controller performs three major functions:

- Identifies the interrupt source.

- Determines interrupt priority.

- Provides the address of the required ISR.

4. Interrupt Priority Levels

The proposed system uses multiple priority levels.

| Priority Level | Interrupt Type | Example |
| --- | --- | --- |
| Level 1 – Highest | Vectored Hardware Interrupt | Emergency device |
| Level 2 – High | Vectored Hardware Interrupt | Real-time sensor |
| Level 3 – Medium | Hardware Interrupt | Communication device |
| Level 4 – Low | Software Interrupt | Background processing |

A higher-priority interrupt can interrupt the execution of a lower-priority interrupt.

5. Vectored Interrupt Handling for High-Priority Devices

High-priority devices are assigned dedicated interrupt vectors.

For example:

| Device | Interrupt Vector |
| --- | --- |
| Emergency Sensor | Vector 10 |
| Real-Time Sensor | Vector 11 |
| High-Speed Device | Vector 12 |
| Communication Device | Vector 20 |

The interrupt flow is:

High-Priority Device

│

▼

Interrupt Request

│

▼

Interrupt Controller

│

▼

Identify Interrupt Vector

│

▼

CPU Directly Jumps to ISR

Vectored interrupts reduce latency because the CPU does not need to check every device to determine the interrupt source.

6. Nested Interrupt Mechanism

Nested interrupts allow a higher-priority interrupt to interrupt a lower-priority ISR.

For example:

CPU Executing Main Program

│

▼

Low-Priority Interrupt

│

▼

Executing Low-Priority ISR

│

│ Higher Priority Interrupt Arrives

▼

High-Priority ISR

│

▼

High-Priority ISR Completed

│

▼

Return to Low-Priority ISR

│

▼

Return to Main Program

Working Process

- A low-priority interrupt occurs.

- The CPU saves the current program state.

- The low-priority ISR begins execution.

- A higher-priority interrupt arrives.

- The interrupt controller compares priorities.

- The higher-priority interrupt is accepted.

- The current ISR state is saved.

- The CPU executes the higher-priority ISR.

- After completion, the CPU restores the previous ISR.

- The interrupted ISR continues execution.

This provides fast service to critical devices.

7. Nested Interrupt Stack

The system uses a stack to store interrupted execution states.

TOP OF STACK

┌─────────────────┐

│ High Priority   │

│ ISR Context     │

├─────────────────┤

│ Low Priority    │

│ ISR Context     │

├─────────────────┤

│ Main Program    │

│ Context         │

└─────────────────┘

Whenever a higher-priority interrupt occurs, the current CPU context is pushed onto the stack.

After the higher-priority ISR finishes, the previous context is restored.

8. Interrupt Latency Analysis

The maximum interrupt latency must satisfy:

Interrupt Latency<10 μs

Interrupt latency can be represented as:

Interrupt Latency=Tdetection+Tpriority+Tvector+Tcontext

Assume:

- Interrupt detection time = 1 µs

- Priority checking time = 1 µs

- Vector identification time = 2 µs

- Context saving time = 3 µs

Therefore:

Interrupt Latency=1+1+2+3
=7 μs

Thus:

7 μs<10 μs

Therefore, the proposed architecture satisfies the maximum interrupt latency requirement.

9. Software Interrupts for Background Tasks

Software interrupts are used for non-critical background activities.

Examples include:

- Logging

- Data analysis

- Device statistics

- Buffer management

- Operating system services

The mechanism is:

Hardware ISR

│

▼

Perform Critical Operation

│

▼

Store Event Information

│

▼

Generate Software Interrupt

│

▼

Background Task Execution

This ensures that lengthy processing does not occur inside a high-priority hardware ISR.

10. Complete Interrupt Handling Flow

Device Event Occurs

│

▼

Hardware Interrupt Generated

│

▼

Interrupt Controller

│

▼

Priority Check

│

├──────── Higher Priority ────────┐

│                                │

▼                                ▼

Vectored Interrupt                 Preempt Current ISR

│                                │

▼                                │

CPU Executes High Priority ISR ◄─────────┘

│

▼

Critical Work Completed

│

▼

Generate Software Interrupt

│

▼

Background Processing

│

▼

Return to Previous Task

11. Preventing Excessive Nesting

Although nested interrupts improve responsiveness, unlimited nesting may create problems such as:

- Stack overflow

- Increased context-switch overhead

- Complex interrupt management

Therefore, the proposed design limits nesting depth.

For example:

Maximum Nesting Depth = 3

The priority system can be:

Level 1 → Can interrupt all lower levels

Level 2 → Can interrupt Levels 3 and 4

Level 3 → Can interrupt Level 4

Level 4 → Cannot interrupt higher levels

This provides controlled nested interrupt handling.

12. Application of Computer Architecture Concepts

The proposed solution applies the following concepts.

Vectored Interrupts

Each high-priority device has a unique vector, allowing direct execution of the correct ISR.

Nested Interrupts

Higher-priority interrupts can preempt lower-priority ISRs.

Priority-Based Interrupt Controller

The interrupt controller compares interrupt priorities before selecting the next ISR.

Context Saving and Restoring

CPU registers and program state are saved before executing an ISR and restored afterward.

Interrupt Stack

The stack stores interrupted program and ISR contexts.

Software Interrupts

Background and non-critical tasks are handled separately from high-priority hardware interrupts.

13. Constraint Satisfaction

| Given Constraint | Proposed Solution |
| --- | --- |
| Support nested interrupts | Priority-based nested interrupt controller |
| Maximum latency < 10 µs | Estimated latency of 7 µs |
| High-priority devices | Dedicated vectored interrupts |
| Background tasks | Software interrupts |
| Fast interrupt identification | Interrupt vector table |
| Priority management | Hardware interrupt controller |

14. Justification

The proposed architecture uses vectored interrupts for high-priority devices because they provide direct access to the required ISR and reduce the time needed to identify the interrupt source.

Nested interrupts allow critical events to preempt lower-priority processing. This ensures that important real-time devices are serviced without waiting for long-running lower-priority ISRs.

The estimated interrupt latency of 7 µs is below the required maximum of 10 µs.

Software interrupts are used for background activities so that non-critical processing does not delay hardware interrupt handling.

A priority-based interrupt controller and controlled nesting depth provide efficient and safe interrupt processing.

15. Final Proposed Architecture

HIGH PRIORITY DEVICE

│

Vectored Interrupt

│

▼

┌──────────────────┐

│ Interrupt        │

│ Controller       │

│                  │

│ Priority + Vector│

└────────┬─────────┘

│

▼

CPU

│

High-Priority ISR

│

Critical Processing

│

▼

Software Interrupt

│

▼

Background Task

│

▼

Return / Resume

16. Conclusion

An interrupt handling mechanism was constructed using priority-based nested interrupts, vectored interrupts, and software interrupts.

High-priority devices use dedicated interrupt vectors, allowing the CPU to quickly locate and execute the correct ISR. Higher-priority interrupts can preempt lower-priority ISRs, providing support for nested interrupt handling.

The estimated interrupt latency is 7 µs, which satisfies the requirement of maintaining latency below 10 µs. Background tasks are processed through software interrupts, ensuring that they do not interfere with critical hardware interrupts.

Therefore, the proposed mechanism provides a fast, efficient, and reliable interrupt handling system that satisfies all the given constraints.

Code of Conduct:

I ___R.MOHANRAM/192521117__ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:mohanram

MARKS ALLOCATION

|  | Problem Understanding(25%) | Constraint Analysis(25%) | Solution Development(25%) | Application of Concepts(15%) | Justification(10%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |