SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2-PSEUDOCODE WRITING TASK (ALGORITHM DESIGN)

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL2-PSEUDOCODE WRITING TASK (ALGORITHM DESIGN) |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | R.MOHANRAM | Reg.No.: | 192521117 |
| Department: | B.Tech IT | Date: | 3-9-26 |

ANNEXURE A

Pseudocode Writing Task (Algorithm Design)

- Write pseudocode to design a DMA-based data transfer algorithm that moves data from an I/O device to memory while handling interrupts and minimizing CPU involvement.

DMA-Based Data Transfer Algorithm

Problem Understanding

Objective: Design a DMA-based algorithm to transfer data from an I/O device to main memory while:

- Minimizing CPU involvement.

- Allowing DMA to perform bulk data transfer.

- Handling transfer-completion interrupts.

- Detecting transfer errors.

Inputs

- IO_DEVICE – Source I/O device.

- MEMORY_ADDRESS – Destination address in main memory.

- DATA_SIZE – Amount of data to transfer.

Output

- Data transferred successfully from the I/O device to memory.

- CPU notified through an interrupt after completion or error.

Pseudocode

ALGORITHM DMA_BASED_DATA_TRANSFER

BEGIN

INPUT IO_DEVICE

INPUT MEMORY_ADDRESS

INPUT DATA_SIZE

// Step 1: Check whether the I/O device is ready

IF IO_DEVICE.STATUS = READY THEN

// Step 2: CPU configures DMA controller

DMA.SOURCE_ADDRESS ← IO_DEVICE.DATA_ADDRESS

DMA.DESTINATION_ADDRESS ← MEMORY_ADDRESS

DMA.TRANSFER_COUNT ← DATA_SIZE

DMA.MODE ← DEVICE_TO_MEMORY

DMA.INTERRUPT_ENABLE ← TRUE

// Step 3: Start DMA transfer

DMA.START ← TRUE

DISPLAY "DMA Transfer Started"

// Step 4: CPU continues executing other tasks

WHILE DMA.STATUS = BUSY DO

CPU.EXECUTE_OTHER_TASKS()

END WHILE

// Step 5: Check for DMA completion

IF DMA.STATUS = COMPLETED THEN

// DMA generates interrupt

GENERATE DMA_INTERRUPT

CALL DMA_INTERRUPT_HANDLER()

ELSE IF DMA.STATUS = ERROR THEN

GENERATE ERROR_INTERRUPT

CALL DMA_ERROR_HANDLER()

END IF

ELSE

DISPLAY "I/O Device Not Ready"

END IF

END

DMA Interrupt Handling Pseudocode

PROCEDURE DMA_INTERRUPT_HANDLER()

BEGIN

// Save CPU state if required

SAVE_CPU_CONTEXT()

// Confirm successful transfer

IF DMA.STATUS = COMPLETED THEN

// Clear DMA completion interrupt

CLEAR DMA_INTERRUPT

DISPLAY "Data Transfer Completed Successfully"

// Notify operating system/application

SET TRANSFER_STATUS ← SUCCESS

END IF

// Restore CPU state

RESTORE_CPU_CONTEXT()

RETURN

END PROCEDURE

DMA Error Handling Pseudocode

PROCEDURE DMA_ERROR_HANDLER()

BEGIN

SAVE_CPU_CONTEXT()

IF DMA.STATUS = ERROR THEN

CLEAR ERROR_INTERRUPT

DISPLAY "DMA Transfer Error"

SET TRANSFER_STATUS ← FAILED

// Stop DMA controller

DMA.START ← FALSE

END IF

RESTORE_CPU_CONTEXT()

RETURN

END PROCEDURE

Algorithm Flow

I/O Device Ready?

│

├── NO ──► Display Error

│

▼ YES

CPU Configures DMA

│

▼

Start DMA Transfer

│

▼

DMA Transfers Data

Directly to Memory

│

▼

CPU Performs Other Tasks

│

▼

Transfer Complete?

│

├── ERROR ──► Error Interrupt

│

▼ YES

Completion Interrupt

│

▼

DMA Interrupt Handler

│

▼

Transfer Successful

Explanation

- The CPU first checks whether the I/O device is ready.

- The CPU configures the DMA controller with the source address, destination address, and data size.

- The DMA controller starts the transfer.

- The DMA controller moves data directly from the I/O device to main memory.

- During the transfer, the CPU performs other tasks, minimizing CPU involvement.

- After completing the transfer, DMA generates an interrupt.

- The interrupt handler confirms completion and notifies the application.

- If an error occurs, an error interrupt handler stops the DMA and reports the failure.

Conclusion

This algorithm minimizes CPU involvement because the CPU only configures the DMA controller and handles the completion interrupt. The actual bulk data transfer is performed directly by the DMA controller, making the mechanism efficient for high-speed I/O communication.

- Develop pseudocode for an interrupt handling system that prioritizes vectored interrupts from multiple devices and dispatches appropriate service routines.

Priority-Based Vectored Interrupt Handling System

Problem Understanding

Objective: Design an interrupt handling algorithm that receives interrupts from multiple devices, prioritizes them, identifies their interrupt vectors, and dispatches the appropriate Interrupt Service Routine (ISR).

Inputs

- INTERRUPT_REQUEST from multiple devices

- PRIORITY of each interrupt

- INTERRUPT_VECTOR of each device

Output

- Highest-priority interrupt is serviced first.

- Correct ISR is executed using the interrupt vector.

- Multiple pending interrupts are handled efficiently.

Main Interrupt Handling Pseudocode

ALGORITHM PRIORITY_VECTORED_INTERRUPT_HANDLER

BEGIN

INITIALIZE INTERRUPT_CONTROLLER

INITIALIZE INTERRUPT_QUEUE

WHILE SYSTEM_IS_RUNNING DO

// Check for incoming interrupts

IF INTERRUPT_CONTROLLER.HAS_PENDING_INTERRUPT() THEN

// Get all pending interrupt requests

PENDING_INTERRUPTS ←

INTERRUPT_CONTROLLER.GET_PENDING_INTERRUPTS()

// Select the highest-priority interrupt

SELECTED_INTERRUPT ←

GET_HIGHEST_PRIORITY(PENDING_INTERRUPTS)

// Obtain interrupt vector

VECTOR ← SELECTED_INTERRUPT.VECTOR_NUMBER

// Identify the corresponding service routine

ISR ← INTERRUPT_VECTOR_TABLE[VECTOR]

// Acknowledge the interrupt

INTERRUPT_CONTROLLER.ACKNOWLEDGE(SELECTED_INTERRUPT)

// Save current CPU state

SAVE_CPU_CONTEXT()

// Dispatch appropriate ISR

CALL ISR()

// Restore CPU state

RESTORE_CPU_CONTEXT()

// Mark interrupt as completed

INTERRUPT_CONTROLLER.COMPLETE(SELECTED_INTERRUPT)

ELSE

// No interrupt is pending

CPU.EXECUTE_NORMAL_PROGRAM()

END IF

END WHILE

END

Procedure to Select the Highest-Priority Interrupt

FUNCTION GET_HIGHEST_PRIORITY(PENDING_INTERRUPTS)

BEGIN

HIGHEST_PRIORITY_INTERRUPT ← NULL

HIGHEST_PRIORITY_VALUE ← -1

FOR EACH INTERRUPT IN PENDING_INTERRUPTS DO

IF INTERRUPT.PRIORITY > HIGHEST_PRIORITY_VALUE THEN

HIGHEST_PRIORITY_VALUE ← INTERRUPT.PRIORITY

HIGHEST_PRIORITY_INTERRUPT ← INTERRUPT

END IF

END FOR

RETURN HIGHEST_PRIORITY_INTERRUPT

END FUNCTION

Vectored Interrupt Dispatch Algorithm

PROCEDURE DISPATCH_VECTORED_INTERRUPT(VECTOR)

BEGIN

// Check whether the vector is valid

IF VECTOR EXISTS IN INTERRUPT_VECTOR_TABLE THEN

// Obtain ISR address

ISR ← INTERRUPT_VECTOR_TABLE[VECTOR]

// Execute corresponding ISR

CALL ISR()

ELSE

// Invalid interrupt vector

CALL INVALID_INTERRUPT_HANDLER()

END IF

END PROCEDURE

Example Interrupt Service Routines

PROCEDURE HIGH_PRIORITY_DEVICE_ISR()

BEGIN

READ DEVICE STATUS

PERFORM CRITICAL OPERATION

CLEAR DEVICE INTERRUPT

RETURN

END PROCEDURE

PROCEDURE MEDIUM_PRIORITY_DEVICE_ISR()

BEGIN

READ DEVICE DATA

STORE DATA IN BUFFER

CLEAR DEVICE INTERRUPT

RETURN

END PROCEDURE

PROCEDURE LOW_PRIORITY_DEVICE_ISR()

BEGIN

RECORD DEVICE EVENT

CLEAR DEVICE INTERRUPT

RETURN

END PROCEDURE

Improved Algorithm with Interrupt Priority Levels

The following priority levels can be used:

| Priority | Device Type | Action |
| --- | --- | --- |
| 1 – Highest | Critical device | Immediate ISR execution |
| 2 – High | Real-time device | Fast service |
| 3 – Medium | Communication device | Normal service |
| 4 – Low | Background device | Service when higher priorities are clear |

Priority Processing Pseudocode

PROCEDURE PROCESS_PENDING_INTERRUPTS()

BEGIN

WHILE INTERRUPT_QUEUE IS NOT EMPTY DO

CURRENT_INTERRUPT ←

REMOVE_HIGHEST_PRIORITY(INTERRUPT_QUEUE)

VECTOR ← CURRENT_INTERRUPT.VECTOR_NUMBER

SAVE_CPU_CONTEXT()

CALL DISPATCH_VECTORED_INTERRUPT(VECTOR)

RESTORE_CPU_CONTEXT()

MARK CURRENT_INTERRUPT AS COMPLETED

END WHILE

END PROCEDURE

Complete Interrupt Flow

Multiple Devices

│

▼

Interrupt Requests Generated

│

▼

Interrupt Controller

│

▼

Check Pending Interrupts

│

▼

Compare Priorities

│

▼

Select Highest Priority

│

▼

Obtain Interrupt Vector

│

▼

Find ISR in Vector Table

│

▼

Save CPU Context

│

▼

Execute Appropriate ISR

│

▼

Clear Interrupt

│

▼

Restore CPU Context

│

▼

Process Next Interrupt

Handling Multiple Interrupts

When multiple devices generate interrupts simultaneously, the algorithm places them in a pending interrupt queue.

IF MULTIPLE INTERRUPTS ARE PENDING THEN

SORT INTERRUPTS BY PRIORITY

WHILE INTERRUPTS ARE PENDING DO

SELECT HIGHEST PRIORITY INTERRUPT

EXECUTE CORRESPONDING ISR

REMOVE INTERRUPT FROM PENDING QUEUE

END WHILE

END IF

For interrupts having the same priority, First-Come, First-Served (FCFS) order can be used.

IF INTERRUPT_A.PRIORITY = INTERRUPT_B.PRIORITY THEN

SELECT INTERRUPT WITH EARLIER ARRIVAL_TIME

END IF

This ensures fair handling among devices with equal priority.

Constraint Handling

| Requirement | Algorithm Solution |
| --- | --- |
| Multiple devices | Maintains pending interrupt list |
| Priority handling | Selects highest-priority interrupt |
| Vectored interrupts | Uses interrupt vector table |
| Correct ISR selection | Vector directly identifies ISR |
| CPU context protection | Saves and restores CPU state |
| Multiple pending interrupts | Processes them using a loop |
| Equal priorities | Uses FCFS ordering |
| Invalid interrupts | Calls error handler |

Conclusion

The proposed algorithm implements a priority-based vectored interrupt handling system. When multiple devices generate interrupts, the interrupt controller identifies all pending requests and selects the highest-priority interrupt.

The interrupt vector is then used to directly locate the correct Interrupt Service Routine (ISR). The CPU context is saved before ISR execution and restored afterward. Remaining interrupts are processed in priority order, while equal-priority interrupts are handled using FCFS.

Therefore, the algorithm provides an efficient, structured, and complete mechanism for handling multiple vectored interrupts from different devices.

- Write pseudocode to select between memory-mapped I/O and I/O-mapped I/O dynamically based on latency and bandwidth requirements.

Dynamic Selection Between Memory-Mapped I/O and I/O-Mapped I/O

Problem Understanding

Objective: Design pseudocode that dynamically selects either Memory-Mapped I/O (MMIO) or I/O-Mapped I/O based on the device's latency and bandwidth requirements.

Inputs

- LATENCY_REQUIREMENT

- BANDWIDTH_REQUIREMENT

- DEVICE_TYPE

- MMIO_AVAILABLE

- IO_MAPPED_AVAILABLE

Output

The algorithm selects the most suitable I/O communication method:

- Memory-Mapped I/O, or

- I/O-Mapped I/O

Selection Criteria

The algorithm uses the following general rules:

| Condition | Selected Method |
| --- | --- |
| High bandwidth required | Memory-Mapped I/O |
| Very low latency required | Memory-Mapped I/O |
| Frequent register access | Memory-Mapped I/O |
| Simple or low-speed device | I/O-Mapped I/O |
| Separate I/O address space required | I/O-Mapped I/O |
| MMIO unavailable | I/O-Mapped I/O |

Main Pseudocode

ALGORITHM SELECT_IO_COMMUNICATION_METHOD

BEGIN

INPUT DEVICE_TYPE

INPUT LATENCY_REQUIREMENT

INPUT BANDWIDTH_REQUIREMENT

INPUT MMIO_AVAILABLE

INPUT IO_MAPPED_AVAILABLE

SET SELECTED_METHOD ← NONE

// Check whether Memory-Mapped I/O is available

IF MMIO_AVAILABLE = TRUE THEN

// Select MMIO for high-performance requirements

IF LATENCY_REQUIREMENT = LOW THEN

SELECTED_METHOD ← MEMORY_MAPPED_IO

ELSE IF BANDWIDTH_REQUIREMENT = HIGH THEN

SELECTED_METHOD ← MEMORY_MAPPED_IO

ELSE IF DEVICE_TYPE = HIGH_SPEED_DEVICE THEN

SELECTED_METHOD ← MEMORY_MAPPED_IO

END IF

END IF

// Select I/O-Mapped I/O if MMIO was not selected

IF SELECTED_METHOD = NONE THEN

IF IO_MAPPED_AVAILABLE = TRUE THEN

SELECTED_METHOD ← IO_MAPPED_IO

ELSE IF MMIO_AVAILABLE = TRUE THEN

// Fallback option

SELECTED_METHOD ← MEMORY_MAPPED_IO

ELSE

SELECTED_METHOD ← NO_IO_METHOD_AVAILABLE

END IF

END IF

DISPLAY SELECTED_METHOD

END

Improved Algorithm Using Numerical Requirements

For a more complete design, numerical latency and bandwidth values can be compared.

Example Thresholds

LOW_LATENCY_THRESHOLD ← 10 microseconds

HIGH_BANDWIDTH_THRESHOLD ← 100 MB/s

Dynamic Selection Pseudocode

ALGORITHM DYNAMIC_IO_SELECTION

BEGIN

INPUT DEVICE_NAME

INPUT REQUIRED_LATENCY

INPUT REQUIRED_BANDWIDTH

INPUT MMIO_AVAILABLE

INPUT IO_MAPPED_AVAILABLE

SET LOW_LATENCY_THRESHOLD ← 10

SET HIGH_BANDWIDTH_THRESHOLD ← 100

SET SELECTED_METHOD ← NONE

// First priority: Low latency requirement

IF REQUIRED_LATENCY <= LOW_LATENCY_THRESHOLD THEN

IF MMIO_AVAILABLE = TRUE THEN

SELECTED_METHOD ← MEMORY_MAPPED_IO

END IF

END IF

// Second priority: High bandwidth requirement

IF SELECTED_METHOD = NONE THEN

IF REQUIRED_BANDWIDTH >= HIGH_BANDWIDTH_THRESHOLD THEN

IF MMIO_AVAILABLE = TRUE THEN

SELECTED_METHOD ← MEMORY_MAPPED_IO

END IF

END IF

END IF

// Third priority: Low-speed or normal device

IF SELECTED_METHOD = NONE THEN

IF IO_MAPPED_AVAILABLE = TRUE THEN

SELECTED_METHOD ← IO_MAPPED_IO

END IF

END IF

// Fallback mechanism

IF SELECTED_METHOD = NONE THEN

IF MMIO_AVAILABLE = TRUE THEN

SELECTED_METHOD ← MEMORY_MAPPED_IO

ELSE

DISPLAY "ERROR: NO I/O METHOD AVAILABLE"

STOP

END IF

END IF

DISPLAY "Device: ", DEVICE_NAME

DISPLAY "Selected I/O Method: ", SELECTED_METHOD

END

Device Communication Procedure

After selecting the I/O method, the system performs communication.

PROCEDURE COMMUNICATE_WITH_DEVICE(SELECTED_METHOD)

BEGIN

IF SELECTED_METHOD = MEMORY_MAPPED_IO THEN

CALL MEMORY_MAPPED_TRANSFER()

ELSE IF SELECTED_METHOD = IO_MAPPED_IO THEN

CALL IO_MAPPED_TRANSFER()

ELSE

DISPLAY "COMMUNICATION FAILED"

END IF

END PROCEDURE

Memory-Mapped I/O Transfer

PROCEDURE MEMORY_MAPPED_TRANSFER()

BEGIN

// Assign device register address

DEVICE_ADDRESS ← GET_MEMORY_ADDRESS()

// Access device using normal memory operations

WRITE DATA TO DEVICE_ADDRESS

RESPONSE ← READ FROM DEVICE_ADDRESS

DISPLAY "Memory-Mapped I/O Transfer Completed"

END PROCEDURE

In Memory-Mapped I/O, device registers are accessed using normal memory instructions.

Example:

WRITE [DEVICE_ADDRESS] ← DATA

DATA ← READ [DEVICE_ADDRESS]

I/O-Mapped I/O Transfer

PROCEDURE IO_MAPPED_TRANSFER()

BEGIN

// Obtain I/O port address

IO_PORT ← GET_IO_PORT_ADDRESS()

// Access device using special I/O instructions

OUTPUT DATA TO IO_PORT

RESPONSE ← INPUT FROM IO_PORT

DISPLAY "I/O-Mapped Transfer Completed"

END PROCEDURE

In I/O-Mapped I/O, a separate I/O address space and special instructions are used.

Complete Dynamic Decision Flow

Start

│

▼

Read Latency Requirement

│

▼

Read Bandwidth Requirement

│

▼

Is Low Latency Required?

│

┌┴─────────────┐

YES            NO

│              │

▼              ▼

MMIO       Check Bandwidth

Available?       │

│                ▼

YES              High Bandwidth?

│               │

▼             ┌─┴─┐

Select         YES  NO

MMIO           │     │

▼     ▼

MMIO   Select

I/O-Mapped

More Complete Version with Device Classification

The following pseudocode also considers the type of device.

FUNCTION SELECT_IO_METHOD

(DEVICE_TYPE, LATENCY, BANDWIDTH)

BEGIN

IF DEVICE_TYPE = NVME_DEVICE THEN

RETURN MEMORY_MAPPED_IO

ELSE IF DEVICE_TYPE = HIGH_SPEED_NETWORK_DEVICE THEN

RETURN MEMORY_MAPPED_IO

ELSE IF DEVICE_TYPE = REAL_TIME_DEVICE THEN

RETURN MEMORY_MAPPED_IO

ELSE IF DEVICE_TYPE = LOW_SPEED_PERIPHERAL THEN

RETURN IO_MAPPED_IO

ELSE

IF LATENCY <= 10 OR BANDWIDTH >= 100 THEN

RETURN MEMORY_MAPPED_IO

ELSE

RETURN IO_MAPPED_IO

END IF

END IF

END FUNCTION

Example Execution

Case 1: High-Speed Device

LATENCY_REQUIREMENT = 5 microseconds

BANDWIDTH_REQUIREMENT = 500 MB/s

The algorithm checks:

5 <= 10

Therefore:

SELECTED_METHOD ← MEMORY_MAPPED_IO

Case 2: Low-Speed Peripheral

LATENCY_REQUIREMENT = 100 microseconds

BANDWIDTH_REQUIREMENT = 10 MB/s

The device does not require very low latency or high bandwidth.

Therefore:

SELECTED_METHOD ← IO_MAPPED_IO

Constraint Handling

| Requirement | Algorithm Solution |
| --- | --- |
| Dynamic selection | Uses conditions based on requirements |
| Low latency | Prioritizes Memory-Mapped I/O |
| High bandwidth | Selects Memory-Mapped I/O |
| Low-speed devices | Can select I/O-Mapped I/O |
| Availability checking | Checks supported I/O methods |
| Error handling | Reports when no method is available |
| Device communication | Calls the appropriate transfer procedure |

Conclusion

The proposed algorithm dynamically selects between Memory-Mapped I/O and I/O-Mapped I/O by analyzing the latency and bandwidth requirements of a device.

When the system requires low latency or high bandwidth, the algorithm selects Memory-Mapped I/O because it allows efficient access through the processor's normal memory address space. For lower-speed or simpler devices where special I/O ports are suitable, the algorithm selects I/O-Mapped I/O.

The algorithm also includes availability checks, fallback conditions, and separate communication procedures, making it a complete and well-structured solution for dynamic I/O communication selection.

- Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating over PCIe, USB, and Thunderbolt interfaces.

Bus Arbitration Algorithm for PCIe, USB, and Thunderbolt Devices

Problem Understanding

Objective: Design pseudocode for a bus arbitration algorithm that manages multiple devices communicating through PCIe, USB, and Thunderbolt interfaces.

The algorithm must:

- Handle multiple device requests.

- Identify the communication interface.

- Select devices fairly.

- Give suitable priority to high-speed devices.

- Avoid starvation.

- Grant bus access to only one selected request at a time.

Inputs

- DEVICE_ID

- INTERFACE_TYPE – PCIe, USB, or Thunderbolt

- PRIORITY

- REQUEST_TIME

- BANDWIDTH_REQUIREMENT

Output

- Selected device receives bus access.

- Data transfer is performed.

- Other devices remain in the waiting queue.

- Fairness is maintained between devices.

Main Bus Arbitration Pseudocode

ALGORITHM BUS_ARBITRATION_SYSTEM

BEGIN

INITIALIZE REQUEST_QUEUE

WHILE SYSTEM_IS_RUNNING DO

// Check for new requests from devices

CALL CHECK_DEVICE_REQUESTS()

// Check whether the communication bus is free

IF BUS.STATUS = FREE THEN

IF REQUEST_QUEUE IS NOT EMPTY THEN

// Select the next device fairly

SELECTED_DEVICE ←

SELECT_NEXT_DEVICE(REQUEST_QUEUE)

// Grant access to selected device

BUS.STATUS ← BUSY

GRANT_BUS_ACCESS(SELECTED_DEVICE)

// Perform data communication

CALL PERFORM_DATA_TRANSFER(SELECTED_DEVICE)

// Release bus after transfer

RELEASE_BUS(SELECTED_DEVICE)

BUS.STATUS ← FREE

END IF

END IF

END WHILE

END

Step 1: Check Device Requests

This procedure checks requests from PCIe, USB, and Thunderbolt devices.

PROCEDURE CHECK_DEVICE_REQUESTS()

BEGIN

FOR EACH DEVICE IN CONNECTED_DEVICES DO

IF DEVICE.REQUEST_BUS = TRUE THEN

// Store device request information

CREATE REQUEST

REQUEST.DEVICE_ID ← DEVICE.ID

REQUEST.INTERFACE_TYPE ← DEVICE.INTERFACE_TYPE

REQUEST.PRIORITY ← DEVICE.PRIORITY

REQUEST.REQUEST_TIME ← CURRENT_TIME

REQUEST.BANDWIDTH ← DEVICE.BANDWIDTH_REQUIREMENT

ADD REQUEST TO REQUEST_QUEUE

END IF

END FOR

END PROCEDURE

Step 2: Select the Next Device

The system uses priority with round-robin fairness.

This means that important high-speed devices can receive suitable priority, but devices that wait for a long time are not permanently blocked.

FUNCTION SELECT_NEXT_DEVICE(REQUEST_QUEUE)

BEGIN

// Update waiting time for all requests

FOR EACH REQUEST IN REQUEST_QUEUE DO

REQUEST.WAITING_TIME ←

CURRENT_TIME - REQUEST.REQUEST_TIME

// Increase priority of devices waiting for a long time

REQUEST.EFFECTIVE_PRIORITY ←

REQUEST.PRIORITY +

CALCULATE_AGING_BONUS(REQUEST.WAITING_TIME)

END FOR

// Find highest effective priority

HIGHEST_PRIORITY ←

FIND_MAX_EFFECTIVE_PRIORITY(REQUEST_QUEUE)

// Select all devices having the same priority

CANDIDATES ←

GET_DEVICES_WITH_PRIORITY(

REQUEST_QUEUE,

HIGHEST_PRIORITY

)

// Apply round-robin selection for fairness

SELECTED_DEVICE ←

ROUND_ROBIN_SELECT(CANDIDATES)

REMOVE SELECTED_DEVICE FROM REQUEST_QUEUE

RETURN SELECTED_DEVICE

END FUNCTION

Step 3: Aging Function to Prevent Starvation

Aging increases the effective priority of devices that wait for a long time.

FUNCTION CALCULATE_AGING_BONUS(WAITING_TIME)

BEGIN

AGING_INTERVAL ← 5

AGING_BONUS ←

WAITING_TIME / AGING_INTERVAL

RETURN AGING_BONUS

END FUNCTION

For example:

DEVICE A

Base Priority = 10

Waiting Time = 2

Effective Priority = 10

DEVICE B

Base Priority = 5

Waiting Time = 30

Effective Priority = 11

Therefore, Device B eventually receives bus access and starvation is prevented.

Step 4: Interface-Based Bus Handling

The selected device is handled according to its communication interface.

PROCEDURE PERFORM_DATA_TRANSFER(SELECTED_DEVICE)

BEGIN

IF SELECTED_DEVICE.INTERFACE_TYPE = PCIE THEN

CALL PCIE_TRANSFER(SELECTED_DEVICE)

ELSE IF SELECTED_DEVICE.INTERFACE_TYPE = USB THEN

CALL USB_TRANSFER(SELECTED_DEVICE)

ELSE IF SELECTED_DEVICE.INTERFACE_TYPE = THUNDERBOLT THEN

CALL THUNDERBOLT_TRANSFER(SELECTED_DEVICE)

ELSE

DISPLAY "UNKNOWN INTERFACE"

END IF

END PROCEDURE

PCIe Communication Procedure

PROCEDURE PCIE_TRANSFER(DEVICE)

BEGIN

DISPLAY "PCIe Device Selected"

ESTABLISH PCIE_LINK

TRANSFER DATA

CHECK TRANSFER_STATUS

IF TRANSFER_STATUS = SUCCESS THEN

DISPLAY "PCIe Transfer Completed"

ELSE

DISPLAY "PCIe Transfer Failed"

END IF

END PROCEDURE

USB Communication Procedure

PROCEDURE USB_TRANSFER(DEVICE)

BEGIN

DISPLAY "USB Device Selected"

CHECK USB_DEVICE_STATUS

IF USB_DEVICE_STATUS = READY THEN

TRANSFER DATA

DISPLAY "USB Transfer Completed"

ELSE

DISPLAY "USB Device Not Ready"

END IF

END PROCEDURE

Thunderbolt Communication Procedure

PROCEDURE THUNDERBOLT_TRANSFER(DEVICE)

BEGIN

DISPLAY "Thunderbolt Device Selected"

ESTABLISH THUNDERBOLT_CONNECTION

TRANSFER HIGH_SPEED_DATA

CHECK TRANSFER_STATUS

IF TRANSFER_STATUS = SUCCESS THEN

DISPLAY "Thunderbolt Transfer Completed"

ELSE

DISPLAY "Thunderbolt Transfer Failed"

END IF

END PROCEDURE

Step 5: Grant Bus Access

PROCEDURE GRANT_BUS_ACCESS(DEVICE)

BEGIN

BUS.GRANTED_TO ← DEVICE.ID

DEVICE.BUS_ACCESS ← TRUE

DISPLAY "Bus Access Granted to ", DEVICE.ID

END PROCEDURE

Step 6: Release Bus

PROCEDURE RELEASE_BUS(DEVICE)

BEGIN

DEVICE.BUS_ACCESS ← FALSE

BUS.GRANTED_TO ← NONE

DISPLAY "Bus Released by ", DEVICE.ID

END PROCEDURE

Complete Simplified Pseudocode

This is the complete algorithm in one flow.

ALGORITHM FAIR_BUS_ARBITRATION

BEGIN

INITIALIZE REQUEST_QUEUE

SET BUS.STATUS ← FREE

WHILE SYSTEM_IS_RUNNING DO

// Receive requests from all devices

FOR EACH DEVICE IN CONNECTED_DEVICES DO

IF DEVICE.REQUEST_BUS = TRUE THEN

ADD DEVICE TO REQUEST_QUEUE

END IF

END FOR

// Check whether bus is available

IF BUS.STATUS = FREE AND

REQUEST_QUEUE IS NOT EMPTY THEN

// Update effective priority using aging

FOR EACH DEVICE IN REQUEST_QUEUE DO

DEVICE.WAITING_TIME ←

CURRENT_TIME -

DEVICE.REQUEST_TIME

DEVICE.EFFECTIVE_PRIORITY ←

DEVICE.PRIORITY +

CALCULATE_AGING_BONUS(

DEVICE.WAITING_TIME

)

END FOR

// Select device

SELECTED_DEVICE ←

SELECT_NEXT_DEVICE(

REQUEST_QUEUE

)

// Grant bus

BUS.STATUS ← BUSY

GRANT_BUS_ACCESS(

SELECTED_DEVICE

)

// Perform transfer based on interface

IF SELECTED_DEVICE.INTERFACE = PCIE THEN

CALL PCIE_TRANSFER(

SELECTED_DEVICE

)

ELSE IF SELECTED_DEVICE.INTERFACE = USB THEN

CALL USB_TRANSFER(

SELECTED_DEVICE

)

ELSE IF SELECTED_DEVICE.INTERFACE =

THUNDERBOLT THEN

CALL THUNDERBOLT_TRANSFER(

SELECTED_DEVICE

)

END IF

// Release bus

RELEASE_BUS(SELECTED_DEVICE)

BUS.STATUS ← FREE

END IF

END WHILE

END

Algorithm Flow

Multiple Devices

PCIe / USB / Thunderbolt

│

▼

Bus Request

│

▼

Request Queue

│

▼

Update Waiting Time

│

▼

Calculate Effective Priority

│

▼

Highest Priority Selected

│

Same Priority?

┌────┴────┐

YES        NO

│          │

▼          ▼

Round-Robin   Select Device

│          │

└────┬─────┘

▼

Grant Bus Access

│

▼

Identify Interface

┌─────┼──────┐

▼     ▼      ▼

PCIe   USB  Thunderbolt

│     │      │

└─────┼──────┘

▼

Data Transfer

│

▼

Release Bus

│

▼

Next Request

Example Device Priority Table

| Device | Interface | Base Priority |
| --- | --- | --- |
| NVMe Storage | PCIe | 10 |
| Graphics Device | PCIe | 9 |
| USB Keyboard | USB | 4 |
| USB Mouse | USB | 5 |
| USB Storage | USB | 6 |
| External Display | Thunderbolt | 9 |
| High-Speed Storage | Thunderbolt | 10 |

The algorithm can prioritize high-performance transfers while aging ensures that lower-priority USB devices eventually receive access.

Constraint Handling

| Requirement | Solution |
| --- | --- |
| Multiple devices | Request queue stores all requests |
| PCIe devices | PCIe transfer procedure |
| USB devices | USB transfer procedure |
| Thunderbolt devices | Thunderbolt transfer procedure |
| Fair bus access | Round-robin selection |
| Prevent starvation | Priority aging |
| Multiple simultaneous requests | Priority comparison |
| Equal-priority requests | Round-robin scheduling |
| Transfer errors | Transfer status checking |

Conclusion

The proposed algorithm manages communication requests from devices using PCIe, USB, and Thunderbolt. All device requests are stored in a request queue, and the algorithm selects devices using effective priority combined with aging.

When devices have equal priority, round-robin arbitration provides fair access. Aging prevents low-priority devices from waiting indefinitely. After a device is selected, the appropriate PCIe, USB, or Thunderbolt communication procedure is executed, and the bus is released for the next request.

Therefore, this algorithm provides a complete, fair, efficient, and scalable bus arbitration mechanism for multiple advanced I/O interfaces.

- Write pseudocode to implement a hybrid I/O communication mechanism that uses polling for low-speed devices and DMA with interrupts for high-speed devices.

Hybrid I/O Communication Mechanism Using Polling and DMA with Interrupts

Problem Understanding

Objective: Design a hybrid I/O communication algorithm that:

- Uses polling for low-speed devices.

- Uses DMA for high-speed devices.

- Uses interrupts to notify the CPU after DMA transfer completion.

- Minimizes unnecessary CPU involvement.

- Selects the correct I/O mechanism based on device speed.

Inputs

- DEVICE_LIST – List of connected devices.

- DEVICE_SPEED – Speed of each device.

- DEVICE_STATUS – READY, BUSY, or ERROR.

- DATA_SIZE – Amount of data to transfer.

Output

- Low-speed devices are serviced using polling.

- High-speed devices transfer data using DMA.

- CPU receives interrupts after high-speed DMA transfers.

- CPU utilization is optimized.

Main Hybrid I/O Algorithm

ALGORITHM HYBRID_IO_COMMUNICATION

BEGIN

INITIALIZE DEVICE_LIST

INITIALIZE DMA_CONTROLLER

INITIALIZE INTERRUPT_CONTROLLER

WHILE SYSTEM_IS_RUNNING DO

FOR EACH DEVICE IN DEVICE_LIST DO

// Check the speed of the device

IF DEVICE.SPEED = LOW_SPEED THEN

CALL POLLING_IO(DEVICE)

ELSE IF DEVICE.SPEED = HIGH_SPEED THEN

CALL DMA_INTERRUPT_IO(DEVICE)

ELSE

DISPLAY "UNKNOWN DEVICE SPEED"

END IF

END FOR

END WHILE

END

Part 1: Polling Algorithm for Low-Speed Devices

Polling is suitable for low-speed devices because checking their status periodically does not create significant CPU overhead.

Examples include:

- Keyboard

- Mouse

- Simple sensors

- Low-speed controllers

PROCEDURE POLLING_IO(DEVICE)

BEGIN

// Check device status

IF DEVICE.STATUS = READY THEN

// Check whether data is available

IF DEVICE.DATA_AVAILABLE = TRUE THEN

// CPU reads a small amount of data

DATA ← READ DEVICE.DATA

STORE DATA IN MEMORY_BUFFER

DISPLAY "Low-Speed Device Data Received"

END IF

ELSE IF DEVICE.STATUS = BUSY THEN

// Device is still processing

CONTINUE

ELSE IF DEVICE.STATUS = ERROR THEN

CALL HANDLE_DEVICE_ERROR(DEVICE)

END IF

END PROCEDURE

Periodic Polling Control

To avoid continuously wasting CPU time, low-speed devices are checked at fixed intervals.

PROCEDURE PERIODIC_POLLING()

BEGIN

SET POLLING_INTERVAL ← FIXED_TIME_INTERVAL

WHILE SYSTEM_IS_RUNNING DO

FOR EACH LOW_SPEED_DEVICE IN LOW_SPEED_DEVICE_LIST DO

CALL POLLING_IO(LOW_SPEED_DEVICE)

END FOR

WAIT FOR POLLING_INTERVAL

END WHILE

END PROCEDURE

This ensures that polling is performed efficiently rather than continuously checking the device.

Part 2: DMA and Interrupt Mechanism for High-Speed Devices

High-speed devices transfer large amounts of data.

Examples include:

- NVMe storage

- High-speed network devices

- Data acquisition devices

- PCIe devices

For these devices, DMA is used to reduce CPU involvement.

PROCEDURE DMA_INTERRUPT_IO(DEVICE)

BEGIN

// Check whether device is ready

IF DEVICE.STATUS = READY THEN

// Check whether data transfer is required

IF DEVICE.DATA_AVAILABLE = TRUE THEN

// Configure DMA controller

DMA.SOURCE_ADDRESS ← DEVICE.DATA_ADDRESS

DMA.DESTINATION_ADDRESS ←

GET_MEMORY_BUFFER_ADDRESS()

DMA.TRANSFER_SIZE ←

DEVICE.DATA_SIZE

DMA.MODE ← DEVICE_TO_MEMORY

DMA.INTERRUPT_ENABLE ← TRUE

// Start DMA transfer

DMA.START ← TRUE

DISPLAY "DMA Transfer Started"

END IF

ELSE IF DEVICE.STATUS = ERROR THEN

CALL HANDLE_DEVICE_ERROR(DEVICE)

END IF

END PROCEDURE

DMA Transfer Completion Interrupt

After the DMA controller completes the data transfer, it generates a hardware interrupt.

PROCEDURE DMA_COMPLETION_INTERRUPT_HANDLER()

BEGIN

// Save current CPU context

SAVE_CPU_CONTEXT()

// Check DMA transfer status

IF DMA.STATUS = COMPLETED THEN

// Clear interrupt

CLEAR DMA_COMPLETION_INTERRUPT

// Mark transfer as complete

DMA.TRANSFER_STATUS ← SUCCESS

DISPLAY "DMA Transfer Completed Successfully"

// Notify application

NOTIFY APPLICATION

ELSE IF DMA.STATUS = ERROR THEN

CALL DMA_ERROR_HANDLER()

END IF

// Restore CPU context

RESTORE_CPU_CONTEXT()

RETURN

END PROCEDURE

DMA Error Handling

PROCEDURE DMA_ERROR_HANDLER()

BEGIN

CLEAR DMA_ERROR_INTERRUPT

DMA.START ← FALSE

DMA.TRANSFER_STATUS ← FAILED

DISPLAY "DMA Transfer Failed"

LOG ERROR

RETURN

END PROCEDURE

Device Classification Algorithm

The following algorithm automatically separates devices into low-speed and high-speed categories.

FUNCTION CLASSIFY_DEVICE(DEVICE)

BEGIN

IF DEVICE.DATA_RATE < SPEED_THRESHOLD THEN

RETURN LOW_SPEED

ELSE

RETURN HIGH_SPEED

END IF

END FUNCTION

Complete Dynamic Hybrid Algorithm

ALGORITHM DYNAMIC_HYBRID_IO_SYSTEM

BEGIN

INITIALIZE DEVICE_LIST

INITIALIZE DMA_CONTROLLER

INITIALIZE INTERRUPT_CONTROLLER

SET SPEED_THRESHOLD ← 100 MB_PER_SECOND

WHILE SYSTEM_IS_RUNNING DO

FOR EACH DEVICE IN DEVICE_LIST DO

DEVICE_TYPE ← CLASSIFY_DEVICE(DEVICE)

// Low-speed devices use polling

IF DEVICE_TYPE = LOW_SPEED THEN

IF TIME_TO_POLL(DEVICE) = TRUE THEN

IF DEVICE.STATUS = READY THEN

IF DEVICE.DATA_AVAILABLE = TRUE THEN

DATA ← READ DEVICE.DATA

STORE DATA IN MEMORY_BUFFER

END IF

ELSE IF DEVICE.STATUS = ERROR THEN

CALL HANDLE_DEVICE_ERROR(DEVICE)

END IF

END IF

// High-speed devices use DMA

ELSE IF DEVICE_TYPE = HIGH_SPEED THEN

IF DEVICE.DATA_AVAILABLE = TRUE AND

DMA.STATUS = IDLE THEN

DMA.SOURCE_ADDRESS ←

DEVICE.DATA_ADDRESS

DMA.DESTINATION_ADDRESS ←

GET_MEMORY_BUFFER_ADDRESS()

DMA.TRANSFER_SIZE ←

DEVICE.DATA_SIZE

DMA.MODE ← DEVICE_TO_MEMORY

DMA.INTERRUPT_ENABLE ← TRUE

DMA.START ← TRUE

END IF

END IF

END FOR

// CPU performs normal processing

CPU.EXECUTE_OTHER_TASKS()

// Check for pending DMA interrupt

IF INTERRUPT_CONTROLLER.HAS_PENDING_INTERRUPT() THEN

INTERRUPT ←

INTERRUPT_CONTROLLER.GET_INTERRUPT()

IF INTERRUPT.TYPE =

DMA_COMPLETION_INTERRUPT THEN

CALL DMA_COMPLETION_INTERRUPT_HANDLER()

ELSE

CALL OTHER_INTERRUPT_HANDLER()

END IF

END IF

END WHILE

END

Hybrid I/O Communication Flow

Connected Devices

│

▼

Check Device Speed

│

┌─────────┴─────────┐

│                   │

▼                   ▼

Low-Speed            High-Speed

Device               Device

│                   │

▼                   ▼

Polling             Configure DMA

│                   │

▼                   ▼

Data Available?       DMA Transfer

│                   │

▼                   ▼

CPU Reads Small       Direct Transfer

Data              to Memory

│                   │

│                   ▼

│             DMA Completed

│                   │

│                   ▼

│              Interrupt

│                   │

└──────────┬────────┘

▼

CPU

│

▼

Continue Processing

Working of the Hybrid System

For Low-Speed Devices

CPU

│

▼

Periodically Check Device

│

▼

Data Available?

│

├── NO ──► Continue

│

▼ YES

Read Small Data

│

▼

Store in Memory

Polling is appropriate because the amount of data and event frequency are low.

For High-Speed Devices

High-Speed Device

│

▼

Data Available

│

▼

CPU Configures DMA

│

▼

DMA Transfers Data

Directly to Memory

│

▼

CPU Executes Other Tasks

│

▼

DMA Completed

│

▼

Interrupt Generated

│

▼

CPU Handles Completion

This minimizes CPU involvement during bulk data transfer.

Comparison of the Two Methods

| Feature | Polling | DMA with Interrupt |
| --- | --- | --- |
| Suitable devices | Low-speed | High-speed |
| CPU involvement | Periodic checking | Very low during transfer |
| Data size | Small | Large |
| Response method | CPU checks status | Device/DMA notifies CPU |
| Transfer method | CPU performs transfer | DMA performs transfer |
| Interrupt overhead | None for normal polling | One completion interrupt |
| Best use | Keyboard, mouse, sensors | NVMe, PCIe, data acquisition |

Constraint Handling

| Requirement | Solution |
| --- | --- |
| Polling for low-speed devices | POLLING_IO() procedure |
| High-speed data transfer | DMA controller |
| CPU involvement minimized | DMA transfers bulk data |
| Completion notification | DMA completion interrupt |
| Multiple devices | FOR EACH DEVICE loop |
| Device errors | Error handling procedure |
| Dynamic selection | Device classification based on speed |
| CPU efficiency | Periodic polling and DMA |

Conclusion

The proposed hybrid I/O communication mechanism combines polling and DMA-based interrupt handling to efficiently manage different types of devices.

Low-speed devices are checked periodically using polling because their data rate is low and polling overhead is manageable. High-speed devices use DMA so that large amounts of data can be transferred directly between the device and main memory without continuous CPU involvement.

After DMA completes the transfer, an interrupt notifies the CPU. This approach reduces unnecessary interrupts for low-speed devices while minimizing CPU involvement for high-speed data transfers.

Therefore, the proposed algorithm provides a complete, efficient, and optimized hybrid I/O communication mechanism that satisfies all the requirements.

Code of Conduct:

I _R.MOHANRAM/192521117_ (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: mohanram

MARKS ALLOCATION

|  | Problem Understanding(20%) | Algorithm Design(30%) | Use of Control Structures(20%) | Pseudocode Structure(10%) | Completeness(20%) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Identifies all inputs, outputs, constraints accurately | Minor missing elements | Partial understanding | Incorrect interpretation |
| Algorithm Design | 30% | Complete, correct, and logically sound solution | Minor logical gaps | Partially correct logic | Incorrect approach |
| Use of Control Structures | 20% | Efficient and appropriate use of loops, conditions, functions/modules | Minor inefficiencies | Limited or basic usage | Incorrect or missing constructs |
| Pseudocode Structure | 10% | Well-structured, clear, consistent format, easy to follow | Mostly clear with minor issues | Difficult to follow in parts | Poorly structured and unclear |
| Completeness | 20% | Handles all cases; optimized approach | Handles most cases; reasonable efficiency | Handles basic cases only | Incomplete or inefficient |